N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                       		Effective Date: Oct. 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION			            Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: 2001 BOOSTER                	Product ID#: 103120                              	HMIS SYMBOL                        HMIS   NFPA       		NFPA SYMBOL

Generic Name: Laundry Booster						HEALTH    |3|   	          SEVERE 4 EXTREME    		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC.  				FLAMMABILITY  |0|         SERIOUS 3 HIGH       		REACTIVITY  |2|

Supplier's Address: 476 Viking Drive, Suite 102  				REACTIVITY    |2| 	   MODERATE 2 MODERATE   	TOXICITY    |3|

Virginia Beach, VA  23452						            SLIGHT 1 SLIGHT         		SPECIAL     |2|

Emergency Phone Number: (800) 255-3924         						         MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                      	CAS #           	WT. %                  	TWA-TLV    	STEL-TLV      	CARCINOGEN

MG/M3               	MG/M3

*Sodium Hydroxide                                 	1310-73-2           	<75                            	2C                 	N/A                          	No

Water                                           	 	7732-18-5         	to 100                     	N/A                 	N/A                          	No

This product contains *toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                     SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212oF       Specific Gravity: 1.3                   			| Flash Point (Method Used): N/A

Vapor Pressure: Like Water    Vapor Density (Air=1): Like Water          		| Upper Explosive Limit: N/A

% Volatile: <5                         pH: >13                                    			| Extinguishing Media: Product is not flammable. Use media appropriate for source

Solubility in Water: Complete Evaporation Rate:   (Water=1): N/A         		| Special Fire Fighting Procedures: Use a self-contained breathing apparatus

Physical Description: Water-thin liquid, characteristic caustic odor.		| Unusual Fire & Explosion Hazards: Contact with some metals can generate H2

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Keep out of reach of children.

Incompatibility (Materials to Avoid): Do not mix with other chemicals.   		| For use by trained personnel only. Keep container closed during storage. For

Hazardous Decomposition Products: Alkaline vapors in a fire.             		| institutional and industrial use only.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Severe skin irritant. Will cause reddening, swelling, and        		| Skin: Flush with water. Seek medical attention if irritation persists.

possible skin damage.                                           				|

Eyes: Severe eye irritant. Liquid and mist may damage eyes, causing   		| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

corneal damage.                                                 				|       attention if irritation persists.

Inhalation: Vapors and mist may be irritating to mucous membranes in  		| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

the throat, and lungs.                                          				|

Ingestion: Irritating and corrosive to the mouth and throat. 			| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

cause headache, nausea, abdominal pain, vomiting and diarrhea. 		|       medical attention immediately.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilation area.                    			| Ventilation: Provide local exhaust to keel TLV of Section 2 ingredients below

|       acceptable limit.

Protective Gloves: Waterproof                                            			| Eye Protection: Safety glasses.

|

Other Protective Equipment: Rubber boots and rubber apron.  			|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Dike and      		| Waste Disposal Method: Any method in accordance to applicable laws..

absorb spill with inert material and transfer to container for disposal. 		|