N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                       		Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 			          Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: 2001 IRON REMOVING SOUR       Product ID#: 103500       	HMIS SYMBOL          	HMIS   NFPA       		NFPA SYMBOL

Generic Name: Iron Remover  						HEALTH        |2|   	           SEVERE 4 EXTREME    		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC.  				FLAMMABILITY  |0|          SERIOUS 3 HIGH       		REACTIVITY  |1|

Supplier's Address: 476 Viking Drive, Suite 102  				REACTIVITY    |1| 	    MODERATE 2 MODERATE   	TOXICITY    |2|

Virginia Beach, VA  23452			 			            SLIGHT 1 SLIGHT        		SPECIAL     |1|

Emergency Phone Number: (800) 255-3924							        MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %             	TWA-TLV        	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

*Oxalic Acid                                       		144-62-7          	<5.0                            1                   	2                           		No

Water                                            		7732-18-5         	to 100                          N/A                 	N/A                          		No

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372:

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                     SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): Like water Specific Gravity: 1.079                    			| Flash Point (Method Used): N/Av

Vapor Pressure: Like water    Vapor Density:      (Air=1): Like water    		| Upper Explosive Limit: N/A

% Volatile: <5                pH: 0.85                                   				| Extinguishing Media: Product is not flammable. Use source-appropriate media.

Solubility in Water: Complete Evaporation Rate:   (Water=1): N/A         		| Special Fire Fighting Procedures: Use caution whenever fighting chemical fires

Physical Description: Light yellow liquid with mild chemical odor.       		| Unusual Fire & Explosion Hazards: only with decomposition, not in normal state

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------                  	SECTION 5 - REACTIVE DATA                             			|        	SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Instable        Hazardous Polymerization: Will not occur   			| Precautions to be Taken in Handling and Storage: Do not get in eyes, skin or

Incompatibility (Materials to Avoid): alkalis producing heat, and silver 		| clothing. Wash thoroughly after handling. Keep container closed when not in use

Hazardous Decomposition Products: Carbon monoxide and formic acid        		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: May produce irritation or corrosive burns to skin. Dilute 			| Skin: Immediately wash with soap and plenty of water while removing

solutions may cause dermatitis after prolonged exposure.                		|          contaminated clothing. Seek medical aid.

Eyes: Can cause irritation or burns.                                  			| Eyes: Immediately flush eyes with plenty of water for at least 15 minutes. Seek

|           medical aid.

Inhalation: Mists may produce respiratory irritation.                 			| Inhalation: Move to fresh air.

Ingestion: Can cause irritation and corrosive burns to mouth throat   		| Ingestion: DO NOT induce vomiting. Give large quantities of water if conscious.

and stomach.                                                            			|                 Seek medical aid.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Where established exposure limits may be exceeded		| Ventilation: Local exhaust is recommended

use a NIOSH-approved container for disposal.

Protective Gloves: rubber 						| Eye Protection: Chemical splash goggles with full-face shield

Other Protective Equipment: Rubber apron is recommended			|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Contain spill 		| Waste Disposal Method: In accordance with Federal, State and Local regulations.

with absorbent material and place in a DOT-approved container for disposal		|

|