Material Safety Data Sheet

- PRODUCT AND COMPANY INFORMATION		DATE:  03/27/2008

REVISION DATE:

Product ID#		200300

Product Name:		Green All Clean

Company Name:	Birsch Industries, Inc.

Address:		476 Viking Drive #102

Va. Beach, VA 23452

Phone:			757-622-0355

Emergency:		800-255-3924

Proper Shipping Name:	Not Regulated

Identification No:	Not Applicable

Hazard Class:		Not Applicable

Generic Description:	Floor Cleaner Concentrate

2.    HAZARDOUS INGREDIENTS

Ingredient:		CAS No				% OF TLV

None Known

3.     PHYSICAL DATA

Boiling Point:		212  F				Specific Gravity:	1.013

Vapor Pressure: 	Not Determined			Percent Volatiles:  	N/A

Vapor Density:		Less than Air			H2O Solubility 		Complete

Evaporation Rate:	Slower than n-Butyl Acetate	pH 			 7.0-8.0

Appearance & Odor:	Clear liquid, Mild Pleasant

4.    FIRE & EXPLOSION DATA

Flash Point:		None

Extinguishing Media:	Foam or carbon dioxide or dry chemical

Firefighting procedure:  	Wear self-contained breathing apparatus with a full face piece operated in pressure-demand or other positive pressure mode when fighting fires.

5.    HEALTH HAZARD DATA

Eyes:			Can cause mild to moderate irritation

Skin:			Prolonged or repeated contact can cause moderate irritation, defatting,

dermatitis.

Breathing:		Excessive inhalation of vapors can cause nasal and respiratory irritation.

Swallowing:		Can cause gastrointestinal irritation, nausea, vomiting, and diarrhea.

First Aid Procedures:

Eyes:  Irrigation of the eye immediately with water for ten (10) minutes is good safety practices.

Skin:  Wash off in flow water or shower.  Wash clothing before reuse.

Inhalation:  Remove to fresh air if effects occur.  If respiration stops, give mouth-to-mouth resuscitation.  Administer oxygen if available.  Call physician and/or transport to medical facility.

Ingestion:  Seek medical attention.  Unless advised otherwise, induce vomiting by giving either syrup of IPECAC followed by 2 glasses of water or by sticking finger down throat.  Do not give anything by mouth if the person is drowsy, unconscious or has no gag reflex.

6.    REACTIVITY DATA

Hazardous Polymerization:  Will not occur

Stability:		Stable

Incompatibility:		Avoid strong oxidizing agents

7.     SPILL & LEAK PROCEDURES

Response to spills:  Stop discharge, if it can be performed safely, and contain material.  If a substantial quantity is spilled, recover with pump or vacuum.  Otherwise, use an absorbent material.  Place contaminated material in a suitable container for disposal.  Do not flush to sewer, stream or other bodies of water.

- PROTECTIVE EQUIPMENT

Respiratory Protection:  If TLV of the product or any component is exceeded, a NIOSH/MSHA jointly approved air supplied respirator is advised.  Engineering or administrative controls should be implemented to reduce exposure.

Ventilation:  Provide sufficient ventilation to maintain exposure below TLV(s)..

Protective gloves:  Wear chemical resistant gloves such as nitrile rubber.

Eye Protection:  Chemical splash goggles are advised.

Other Protective Equipment:  Wear impervious clothing and boots.

- SPECIAL PRECAUTIONS

Precautions to be taken in Handling and Storage:  Handle with reasonable care.  Avoid breathing vapors.  Store in a cool, dry place.  Dispose of empty containers in accordance with local, state and federal guidelines.