# N/A = Not Applicable                                      	       	MATERIAL SAFETY DATA SHEET                           	   Effective Date: April  2005

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BOWL & RESTROOM CLEANER                            	HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Restroom Cleaner Acid Replacement  Product ID#: 172467     	HEALTH        |1|   	             SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 			FLAMMABILITY  |0|                SERIOUS 3 HIGH      		REACTIVITY  |1|

Supplier's Address: 476 VIKING DRIVE				REACTIVITY    |1|            MODERATE 2 MODERATE  	TOXICITY    |1|

VIRGINIA BEACH, VA 23452                                      	SLIGHT 1 SLIGHT    				SPECIAL     |1|

Emergency Phone Number: (800) 255-3924                                                                        	      	     MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                	TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Water                                             		7732-18-5        	to 100

PROPRIETARY			Proprietary		< 5.0							No

ALCOHOL ETHOXYLATE		68439-46-3	< 2.0						No

PROPRIETARY			Proprietary		< 1.0							No

*This product contains  toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                			|                      SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212o      		Specific Gravity: 1.00                     	| Flash Point (Method Used): N/A

Vapor Pressure: 20 mm Hg@68oF  	Vapor Density:      (Air=1): >1            	| Upper Explosive Limit: N/A

% Volatile: 90                		pH: 1.0                                    		| Extinguishing Media: N/A

Solubility in Water: Complete 		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: N/A

Physical Description: Blue watery thin liquid.        				| Unusual Fire & Explosion Hazards: Reacts with metals to release Hydrogen.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|        	 SECTION 6 - STORAGE AND HANDLING INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      			| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid): Avoid metals and other chemicals   		| 	over three high. Avoid inhalation of vapors or mists. Avoid all personal

Hazardous Decomposition Products: Do not use on aluminum or soft metals. 		| 	contact.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  				| First Aid Procedures

Primary Route of Entry:						|

Skin: Prolonged or repeated contact can cause tissue damage.                                       	| Skin: Wash exposed area with soap and water for at least 15 minutes, while

|	removing clothing. Call physician. Wash clothes before reuse.

Eyes: Eye burns on contact                                            				| Eyes: Flush thoroughly with water for at least 15 minutes. Get medical aid. If

|	medical aid is not available, flush an additional 15 minutes.

Inhalation: Not a likely route of exposure due to physical properties.   		| Inhalation: Remove from further exposure. If unconsciousness occurs, seek

and/or vapors.                                                          			|	immediate medical aid. Breathing stops - use mouth-to-mouth resuscitation

Ingestion: Do not take internally. Product may be harmful or fatal if ingested.        	| Ingestion: DO NOT induce vomiting. If conscious, give large quantities of milk

|	or water. Get medical assistance immediately.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Approved respirator, if vapors or mists exceed   		| Ventilation: No special requirements under ordinary conditions.

applicable standards.						|

Protective Gloves: Rubber                                                				| Eye Protection: Goggles or face shield.

Other Protective Equipment: Rubber apron and    				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN rubber boots.

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Wear protect- 		| Waste Disposal Method: At appropriate facility in accordance with applicable

ive clothing. Sprinkle hydrated lime or soda ash. Scrape up and remove.  		| laws and regulations.

Small - dilute with water and mop up. Wear gloves and goggles always.    		|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------