N/A = Not Applicable                                      	      MATERIAL SAFETY DATA SHEET                                       	Effective Date: Oct 2002

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION			  Revised Date: Feb 2004

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BRIGHT DESTAINER BRIGHTENER      Product ID#: 213200              	HMIS SYMBOL                        HMIS     NFPA       	      NFPA SYMBOL

Generic Name: Laundry Brightener                                				HEALTH    |2|   	           SEVERE 4 EXTREME    	      FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC.  				FLAMMABILITY  |0|         SERIOUS 3 HIGH       	      REACTIVITY  |2|

Supplier's Address: 476 Viking Drive, Ste 102  				REACTIVITY    |2|        MODERATE 2 MODERATE       TOXICITY    |2|

Virginia Beach, VA  23452			            			            SLIGHT 1 SLIGHT 	      SPECIAL     |0|

Emergency Phone Number: (800) 255-3924							        MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                           	CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

*Hydrogen Peroxide                                 	7722-84-1         	<15.0                          1.4                 	N/A                          	No

Water                                           	  	7732-18-5        	to 100                          N/A                 	N/A                          	No

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                     SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): like water Specific Gravity: 1.035                    			| Flash Point (Method Used): N/A

Vapor Pressure: like water    Vapor Density:      (Air=1): like water    		| Upper Explosive Limit: N/A

% Volatile: <5                pH: 5.4                                    				| Extinguishing Media: Product is not flammable or combustible

Solubility in Water: Complete Evaporation Rate:   (Water=1): N/A         		| Special Fire Fighting Procedures: Self-contained breathing apparatus

Physical Description: Colorless liquid, slightly pungent odor            		| Unusual Fire & Explosion Hazards: May release oxygen if exposed to heat

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|                 SECTION 6 - STORAGE AND HANDLING INFORMATION

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Unstable (heat) Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Store in a cool place.

Incompatibility (Materials to Avoid): Acids, combustibles, and oxidizers.    		| contamination from any source, inc. metals or organic materials.

Hazardous Decomposition Products: releases O2, increases burning rate.    		|

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: May cause chemical burns - solutions are corrosive. Effects     		| Skin: In case of contact, wash with soap and plenty of water for at least 15

range from irritation to blistering and necrosis.                       			|          minutes while removing contaminated clothing. Seek medical aid.

Eyes: May cause chemical burns. May result in corneal injury and      		| Eyes: Immediately flush with plenty of water for at least 15 minutes. Seek

blindness.                                                              				|           medical aid.

Inhalation: May cause irritation or chemical burns to mucous membranes		| Inhalation: Remove to fresh air. Seek medical aid.

and respiratory system.						|

Ingestion: May cause chemical burns. Decomposition will release O2, water.	| Ingestion: DO NOT induce vomiting. If conscious, give large quantities of

may distend the esophagus or stomach and cause severe damage.           		|                  Sit upright. Call a physician.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Not expected to be necessary. In emergency use  		| Ventilation: Local exhaust

NIOSH app. respirator.						|

Protective Gloves: Rubber                                                			| Eye Protection: Chemical splash goggles or full-face shield.

Other Protective Equipment: Rubber apron

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN  THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Dike area.    		| Waste Disposal Method: In accordance with all Federal, State and Local

Absorb on non-combustible material and place in DOT-approved 	   		| regulations.

for disposal. Flush spill site with water to sewer.                      			|