N/A = Not Applicable                                		 MATERIAL SAFETY DATA SHEET  		                      	Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION			         Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BRITE 4:1 WINDOW CLEANER         Product ID#: 107410          HMIS SYMBOL     	     	HMIS   NFPA      	 	 NFPA SYMBOL

Generic Name: Window Cleaner                   				HEALTH        |1|    	          SEVERE 4 EXTREME   	 FIRE        |1|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |1|        SERIOUS 3 HIGH      		 REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0|       MODERATE 2 MODERATE  	 TOXICITY    |1|

Virginia Beach, VA  23452					                           SLIGHT 1 SLIGHT    		 SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                                   MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        TWA-TLV             STEL-TLV          	CARCINOGEN

MG/M3               	MG/M3

Nonionic Detergent                               		26027-38-3         	< 1.0                         N/A                 	N/A                          No

Isopropyl Alcohol                                   		67-63-0         	<20.0                        983                 	1230                         No

Ethylene glycol monobutyl ether                    	111-76-2         	< 6.0                         121                 	N/A                          No

Ammonium Hydroxide                                	1336-21-6         	< 1.0                         N/A                 	N/A                          No

Water                                             		7732-18-5         	to 100                       N/A                 	N/A                          No

This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              	|                     SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212o         Specific Gravity: 0.96                  		| Flash Point (Method Used): NONE

Vapor Pressure: 20 mm Hg @68 F   Vapor Density:   (Air=1): >1            	| Upper Explosive Limit: 12              Lower Explosive Limit: 2

% Volatile: 98+ pH: 7.5                                 				| Extinguishing Media: Water or CO2

Solubility in Water: Complete    Evaporation Rate: (Water=1): >1         		| Special Fire Fighting Procedures: None

Physical Description: Blue, thin liquid with a strong ammonia odor.      		| Unusual Fire & Explosion Hazards: None

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                  SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid):                                    			| over three high.

Hazardous Decomposition Products:                                        		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Prolonged or repeated contact may cause dermatitis.             		| Skin: Flush with water. Seek medical attention if irritation persists.

Eyes: Eye irritant. Liquid and mist may damage the eyes, causing      	| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

corneal damage.                                                 			|       attention.

Inhalation: Vapors may be irritating to the mucous membranes of       	| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

the nose, throat, and lungs.                                    			|

Ingestion: May be irritating to the mouth, throat, and gastrointestinal-  	| Ingestion: Dilute with 2 glasses of water. Seek medical attention if

system. Vomiting and diarrhea expected with large doses         		|       symptoms persist.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: No special requirements.                         		| Ventilation: Provide local exhaust to keep TLV of Section 2 ingredients

|	below acceptable limit.

Protective Gloves: Waterproof recommended.                               		| Eye Protection: Safety glasses recommended.

Other Protective Equipment: N/A

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Extinguish all 	| Waste Disposal Method: Preferred disposal is reuse in a process. Alternatively,

sources of ignition. Squeegee or mop into drain. Flush with water. For   	| disposal in an environmentally secure landfill with proper environmental larger

quantities, use absorbent material such as clay or sawdust.              		| permits is recommended.