N/A = Not Applicable                                      			MATERIAL SAFETY DATA SHEET                                       	Effective Date: Oct 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 		              Revised Date: Feb 2004

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BRITE 40:1 			Product ID#: 107450		HMIS SYMBOL           	HMIS   NFPA       		NFPA SYMBOL

Generic Name: WINDOW CLEANER 					HEALTH    |1|   	             SEVERE 4 EXTREME    	FIRE        |2|

Supplier's Name: BIRSCH INDUSTRIES, INC.  				FLAMMABILITY  |2|           SERIOUS 3 HIGH       		REACTIVITY  |0|

Supplier’s Address: 476 Viking Drive, Suite 102  				REACTIVITY    |0| 	      MODERATE 2 MODERATE   	TOXICITY    |1|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT         	SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						          	          MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                      	CAS #           	WT. %                  	TWA-TLV    	STEL-TLV      	CARCINOGEN

MG/M3               	MG/M3

*Isopropyl Alcohol 			67-63-0		<13.0		983		1230		No                                                                                               *Ethylene Glycol Monobutyl Ether	111-76-2		<6.0		121		N/A		No

*Ammonium Hydroxide		1336-21-6		<6.0		N/A		N/A		No

Nonionic Detergent			26027-38-3	<2.0		N/A		N/A		No

Water                                            		7732-18-5         	to 100                     	N/A                 	N/A                          	No

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                     SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212oF     Specific Gravity: 0.96                     			| Flash Point (Method Used): >120 oF

Vapor Pressure: 20 mm Hg @68oF    Vapor Density (Air=1): >1          		| Upper Explosive Limit: 10	Lower Explosive Limits: 4

% Volatile: 98*                pH: >7.5                                    			| Extinguishing Media: Any

Solubility in Water: Complete Evaporation Rate:   (Water=1): >1         		| Special Fire Fighting Procedures: Water is very effective

Physical Description: Blue, thin liquid with a strong ammonia odor. 		| Unusual Fire & Explosion Hazards: Drums heated by fire can explode

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	SECTION 6 - STORAGE AND HANDLING INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Keep out of reach of children.

Incompatibility (Materials to Avoid): Strong Oxidizers		   		| For use by trained personnel only. Keep container closed during storage. For

Hazardous Decomposition Products: Carbon monoxide possible if burned.             	| institutional and industrial use only.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Prolonged or repeated contact may cause dermatitis.			| Skin: Flush with water. Seek medical attention if irritation persists.

|

Eyes: Eye irritant. Liquid and mist may damage eyes, causing   			| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

corneal damage.                                                 				|       attention.

Inhalation: Vapors and mist may be irritating to mucous membranes in  		| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

the nose, throat, and lungs.                                          			|

Ingestion: May be irritating to the mouth, throat and gastrointestinal system. 		| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

Vomiting and diarrhea expected with large doses. 				|       medical attention immediately.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: No special requirements.                    			| Ventilation: Provide local exhaust to keel TLV of Section 2 ingredients below

|       acceptable limit.

Protective Gloves: Waterproof recommended                                            		| Eye Protection: Safety glasses recommended.

|

Other Protective Equipment: N/A  					|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Extinguish all 		| Waste Disposal Method: Any method in accordance to applicable laws.

sources of ignition.  Squeegee or mop into drain.  Flush with water			|

For, quantities use absorbent material such as clay or sawdust.      			|

absorb spill with inert material and transfer to container for disposal. 		|