N/A = Not Applicable                                      			MATERIAL SAFETY DATA SHEET                                       	Effective Date: May 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BRITE AEROSOL GLASS CLEANER           Product ID#: 2006200         HMIS SYMBOL           	HMIS   NFPA       		NFPA SYMBOL

Generic Name: GLASS AND WINDOW CLEANER 				HEALTH    |1|   	             SEVERE 4 EXTREME    	FIRE        |2|

Supplier's Name: BIRSCH INDUSTRIES, INC.  				FLAMMABILITY  |2|           SERIOUS 3 HIGH       		REACTIVITY  |1|

Supplier's Address: 476 Viking Drive, Suite 102  				REACTIVITY    |1| 	      MODERATE 2 MODERATE   	TOXICITY    |1|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT         	SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						                            MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                      	CAS #           	WT. %                  	TWA-TLV    	STEL-TLV      	CARCINOGEN

MG/M3               	MG/M3

2-Propanol				67-63-0		5-10		400 PPM		N/A		No

*2-Butoxyethanol			111-76-2		1-5		500 PPM		N/A		No

Butane				106-97-8		1-5		800 PPM		N/A		No

Propane                                            	74-98-6         	1-5                     	1000 PPM                 	N/A                          	No

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372:

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                     SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >200oF     Specific Gravity: 0.98                     			| Flash Point (Method Used): >101 oF

Vapor Pressure: 20 mm Hg @68oF    Vapor Density (Air=1): <1          		| Upper Explosive Limit: 12	Lower Explosive Limits: 2

% Volatile: N/A                pH: N/A                                    			| Extinguishing Media: Water, CO2

Solubility in Water: Complete Evaporation Rate:   (Water=1): <1         		| Special Fire Fighting Procedures: KEEP CONTAINERS COOL.  Protect Personnel.

Physical Description: Clear, water white liquid with a pleasant scent. 		| Unusual Fire & Explosion Hazards: Containers may vent or rupture when hot.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	SECTION 6 - STORAGE AND HANDLING INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: CAUTION- CONTENTS UNDER

Incompatibility (Materials to Avoid): None		   			| PRESSURE.  KEEP OUT OF REACH OF CHILDREN.  STORE IN A COOL PLACE

Hazardous Decomposition Products: Oxides of carbon form upon combustion.             	| (<120o), DRY AREA.  AVOID CONTACT WITH EYES AND SKIN.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  				| First Aid Procedures

Primary Route of Entry:						|

Skin: Prolonged or repeated contact may cause irritation.			| Skin: Flush with water. Seek medical attention if irritation persists.

|

Eyes: May cause transient pain.						| Eyes: Flush with large quantities of water for at least 15 minutes,  holding

|	eyelids open. Seek medical attention.

Inhalation: Prolonged or repeated inhalation may cause irritation.  		| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

|

Ingestion: Do not take internally			 		| Ingestion: Seek medical attention

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: No special requirements.                    			| Ventilation: Provide local exhaust to keel TLV of Section 2 ingredients below

|       acceptable limit.

Protective Gloves: Plastic						| Eye Protection: Safety glasses recommended.

|

Other Protective Equipment: N/A  					|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Extinguish all 		| Waste Disposal Method: Replace cap and discard container in trash.

sources of ignition.  Provide  adequate ventilation.  Wipe up spill.			|  DO NOT puncture or incinerate.