N/A = Not Applicable                                      	        MATERIAL SAFETY DATA SHEET                                       	Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION  				Revised SEPT 2005

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BROWN-X STAIN REMOVER		Product ID#:105040		HMIS SYMBOL     		    HMIS   NFPA      		NFPA SYMBOL

Generic Name: STAIN REMOVER                                			HEALTH        |1|   	                SEVERE 4 EXTREME      	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|               SERIOUS 3 HIGH    		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0|              MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452							SLIGHT 1 SLIGHT    		SPECIAL     |0 |

Emergency Phone Number: (800) 255-3924                                                                                                                         MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %       	PEL   	TWA-TLV      	STEL-TLV    	CARCINOGEN

Water                                              		7732-18-5         	to 100     		N.E.           N.E.                 	N.E.                       	NO

Hydroxyacedic Acid                                   		79-14-1          	5-10      		N.E.           N.E.                                            		NO

Isopropanol                                          		67-63-0          	1-5      		400ppm 	400ppm

Nonionic Surfactant                                               	1-5       		N.E.      		N.E.

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              	|                          SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 210-220oF  		Specific Gravity: 1.02                      	| Flash Point (Method Used): N.A.

Vapor Pressure: <37           		Vapor Density: N.D. (Air=1): 1             	| Upper Explosive Limit: N.A.

% Volatile:                   		pH: 2.2                                    		| Extinguishing Media: N/A

Solubility in Water: yes      		Evaporation Rate (Water=1): <1             	| Special Fire Fighting Procedures: Not flammable

Physical Description:                                                    				| Unusual Fire & Explosion Hazards: None known

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                            	|        	  SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable                Hazardous Polymerization: Will not occur.                	| Precautions to be Taken in Handling and Storage: Use goor personal hygiene. Wash

Incompatibility (Materials to Avoid): Strong oxidizers and bases.                            	|contaminated clothing and equipment before reuse.

Hazardous Decomposition Products:  None known                                      		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Preexisting disorder may be aggravated.                                                                	| Skin: Flush with water. Call a physician if irritation persists.

|

Eyes: Brief mild irritation. Preexisting disorder may be aggravated.                            	| Eyes: Flush with water for at least 15 minutes. Call a physician.

|

Inhalation:  Mist may irritate respiratory tract. May aggravate existing condition.   	| Inhalation: Remove to fresh air.

|

Ingestion: Preexisting disorder may be aggravated.                                                       	| Ingestion: DO NOT induce vomiting. Drink large quantities of water. Call a physician.

|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: None required with proper room ventilation.                        	| Ventilation: Maintain good ventilation.

|

Protective Gloves: Rubber.                                                      			| Eye Protection: Safety goggles if splashing is likely.

|

Other Protective Equipment:  None KEEP OUT OF THE REACH OF CHILDREN.	|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Use mop or wet vacuum    	| Waste Disposal Method: In accordance with all State, Federal and local regulations.

to collect material for proper disposal. Rinse area with water		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------