N/A = Not Applicable                                      		 MATERIAL SAFETY DATA SHEET                                     Effective Date: OCT 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION 			           Revised Date: MARCH 2008

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: BRIGHT CHLORINE BLEACH     Product  ID#: 211050            	HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Laundry Bleach:                                				HEALTH        |4|   	            SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|           SERIOUS 3 HIGH      		REACTIVITY  |4|

Supplier's Address: 476 Viking Drive, Ste 102 				REACTIVITY    |4|         MODERATE 2 MODERATE  	TOXICITY    |4|

Virginia Beach, VA  23452		           				             SLIGHT 1 SLIGHT    		SPECIAL       |1|

Emergency Phone Number: (800) 255-3924						   	         MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       CAS #           	WT. %                        	TWA-TLV             	STEL-TLV                  	CARCINOGEN

MG/M3               	MG/M3

*Sodium Hypochlorite                               	7681-52-9         	<13.0                          	N/A                 	N/A                          		No

Water                                           	  	7732-18-5        	to 100                          	N/A                 	N/A                          		No

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              	|                         SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212o F     		Specific Gravity: 1.14-1.165               	| Flash Point (Method Used): N/A

Vapor Pressure: Like water    		Vapor Density:      (Air=1): Like water    	| Upper Explosive Limit: N/A

% Volatile: <10               		pH: 12.7                                   		| Extinguishing Media: Product is not flammable.

Solubility in Water: Complete 		Evaporation Rate:   (Water=1): N/Av        	| Special Fire Fighting Procedures: Use self-contained breathing apparatus.

Physical Description: Clear pale yellow liquid, chlorine odor            			| Unusual Fire & Explosion Hazards: None known

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                        SECTION 6 - STORAGE AND HANDLING INFORMATION

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Unstable        Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Keep container closed. Wash

Incompatibility (Materials to Avoid): Mixtures with acids or ammonia     		| 	thoroughly after handling.

Hazardous Decomposition Products: Chlorine gas                           			|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: According to DOT, skin testing on rabbits with a 9% and 12% sol.		| Skin: Wash with plenty of water for at least 15 minutes. Call physician if

showed erythema and adema, but no signs of corrosivity.                      		|	irritation persists.

Eyes: Severe irritation or burns.                                     			| Eyes: Immediately flush with plenty of water for at least 15 minutes. Seek

|	medical attention.

Inhalation: Irritation to mucous membranes. If mixed with an acid,    		| Inhalation: Remove to fresh air. If irritation persists, seek medical aid.

chlorine gas is released (extremely irritating to respiratory tract).			|

Ingestion: Irritation of mouth, throat and stomach with pain and      		| Ingestion: DO NOT induce vomiting. If conscious, give large quantities of milk

possible ulceration. Esophageal strictures occur rarely. 			|	or water. Call a physician.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Not required.                                    			| Ventilation: Local exhaust ventilation recommended where misting occurs.

|

Protective Gloves: Rubber                                                			| Eye Protection: Chemical splash goggles or face shield.

|

Other Protective Equipment: Rubber apron where concentrated 			|

product may be splashed						|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Neutralize    		| Waste Disposal Method: In accordance with all Federal, Sate and Local

with soda ash and water. Transfer slurry to container for disposal.      		| 	regulations.

|

US DOT INFORMATION

HYPOCHLORITE SOLUTIONS

8, UN1791, PGIII