N/A = Not Applicable                                      		  MATERIAL SAFETY DATA SHEET                                                             Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION				  Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name:  CITRA CLEAN			Product ID#:109470		HMIS SYMBOL     		HMIS   NFPA     		NFPA SYMBOL

Generic Name: DEGREASER/DEODORANT 				HEALTH        |1|   	           SEVERE 4 EXTREME   		FIRE        |2|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |2|           SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0|          MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                                                     MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

SAFE HANDLING PROCEDURES: Precautions To Be Taken In Handling and Storage: Usual precautions for combustible liquids. Keep temperatures below 140oF. Store

in tightly sealed, full containers. Clean up all spills. All handling equipment should be grounded. Product may expand slightly in storage causing

pressure to build on container. Open container carefully if product appears to be under pressure. Wash thoroughly after handling.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT. %                        TWA-TLV             	STEL-TLV                 	CARCINOGEN

MG/M3               	MG/M3

Water						7732-18-5		to 100		N/A		N/A		No

Citrus Terpenes                                   			5989-27-5         	90-100                         	           		 	No

Coconut Diethanolamine				8051-30-7		0-5						No

Potassium Oleate					0143-18-0		0 to 5						No

Nonionic Surfactant                              			26027-38-3          	5-10                                                                           		No

This product contains no hazardous compounds as defined by OSHA Regulations (29 CFR 19 10). Product is a by-product of citrus, entirely of natural origin,

and believed to contain no artificial flavors, sulfites, nitrites or pesticides residue exceeding tolerances established by the FDA.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                         		|                         SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 300oF            		Specific Gravity: 0.97               		| Flash Point (Method Used): >200oF (TCC)

Vapor Pressure:<10mm Hg @68F        	Vapor Density: (Air=1) >1            	| Upper Explosive Limit: N/A

% Volatile: 90+                     		pH: 11.5 to 12.0                     		| Extinguishing Media: CO2, Dry Chemical, Foam.

Solubility in Water: Emulsifies     		Evaporation Rate:(Ether=1) <1    | Special Fire Fighting Procedures: Do not use water.

Physical Description: Slightly viscous liquid with pleasant odor.			| Unusual Fire & Explosion Hazards: Drums heated by fire can explode.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                     SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          		Hazardous Polymerization: None                	| Precautions to be Taken in Handling and Storage: For use by trained personnel

Incompatibility (Materials to Avoid): None known				| 	only. Keep container closed during storage. Keep out of reach of children.

Hazardous Decomposition Products: Carbon monoxide possible if burned.    		| 	For institutional and industrial use only.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:		    				|

Skin: Defatting to skin with prolonged or repeated use.  May lead to dryness             	| Skin: Wash thoroughly with water. If irritated persists, seek medical

and irritation.                                                                         		|       	attention.

Eyes: Eye irritant. May cause redness, burning, or tearing.           			| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

|       	attention if irritation persists.

Inhalation: Vapors and mists may be irritating to mucous membranes    		| Inhalation: Remove to fresh air. Seek medical attention if irritation persists.

in the nose, throat, and lungs.                                 			|

Ingestion: May be irritating to the mouth, throat, and                			| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

gastrointestinal system.                                        			|       	medical attention immediately.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilated area.                     			| Ventilation: Provide local exhaust to keep TLV of Section 2 ingredients

|        	below acceptable limit.

Protective Gloves: Solvent proof.                                        			| Eye Protection: Safety glasses.

Other Protective Equipment: N/A        					|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Mop up or  		| Waste Disposal Method: Small spills - flush into sewer system. Large - contain

otherwise absorb and hold for disposal.				| with absorbent material and discharge in accordance with all Federal, State and

| Local regulations. CAUTION: Slippery on floor.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------