N/A = Not Applicable                                                 MATERIAL SAFETY DATA SHEET                                              Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 				 Revised Date: Feb. 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: CLASSIC 22		 	Product ID#: 109660		HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Wax/Finish 						HEALTH        |1|   	            SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|           SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102				REACTIVITY    |0| 	     MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                                                     MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT.	------ Exposure Limits ------ 	      VP

ACGIG/TLV       	OSHA/PEL    	mm HG

DIETHYLENE GLYCOL METHYL ETHER		00011-77-3	<58%	.2000 MMHG @ 680 F	     NONE

*  These items are listed on the SARA TITLE III Section 313 inventory.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                  	                 |                          SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212oF       		Specific Gravity: 1.03	                    	| Flash Point (Method Used): None

Vapor Pressure: N/A           		Vapor Density:      (Air=1): N/A           	| Upper Explosive Limit: N/A

% Volatile: 44-46  %          		pH: 8.0-8.5                                    	| Extinguishing Media: CO2, Water Mist, Dry Chemical, Foam

Solubility in Water: Yes      		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: None

Physical Description: Milky white emulsion with mild odor. 	 		| Unusual Fire & Explosion Hazards: None

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	     SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not store or stack over

Incompatibility (Materials to Avoid): None                               			| three cases or pails high.

Hazardous Decomposition Products: None                                   			|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Eye or ingestion					|

Skin: Redness and drying                                              			| Skin: Wash thoroughly with water. If irritation persists, call physician.

Eyes: Discomfort, redness, and moderate irritation                    			| Eyes: Flush with water for at least 15 minutes. Call physician if irritation

|	continues.

Inhalation: N/A                                                       				| Inhalation: None

Ingestion: Slight mucosal damage along with stomach cramps, nausea    		| Ingestion: Immediately drink large quantities of water. Contact a physician.

and/or vomiting.						|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: None required                                    			| Ventilation: Local exhaust

Protective Gloves: None                                                  			| Eye Protection: Protective goggles should be worn

Other Protective Equipment: None required

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Wipe up with  		| Waste Disposal Method: Small spills may be flushed into sewer systems. Large

damp mop, sponge or rags, and discard. Flush area with clean water if    		| spills should be contained with absorbent material and discarded in accordance

necessary. Avoid discharge into open waterways.                          			| with all Federal, State and Local regulations.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------