N/A = Not Applicable                                      	       	MATERIAL SAFETY DATA SHEET                           	   Effective Date: Oct 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION			  Revised Date: Sept 2005

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: Classic Floor Wax                     Product ID#: 109990       	HMIS SYMBOL     		  HMIS    NFPA      		NFPA SYMBOL

Generic Name: Wax                                                                 		HEALTH        |1|   	              SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 			FLAMMABILITY  |0|             SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 			REACTIVITY    |0|            MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452					               SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                        	           MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                	-----------Exposure Limits------------------ 	VP

ACGIG/TLV	OSHA/PEL	mmHG

*Aqua Ammonia                                   		1336-21-6         	1.0		25 (NH3)	                  35 (NH3)                        115

*deg methyl ether                                     		111-77-3		3.0		NE		NE		    .3

*dibutyl  phthalate                                            	84-74-2       	1.5		5mg/M3		5mg/M3		     1

*These items are listed on the SARA TITLE III Section 313 inventory

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                		|                      SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >210o      		Specific Gravity: 1.005                     	| Flash Point (Method Used): N/A

Vapor Pressure: N/A 			Vapor Density:      (Air=1): N/A            	| Upper Explosive Limit: N/A

% Volatile: 44-46	                		pH: 8.6                                    		| Extinguishing Media: CO2 Water Mist, Dry Chemical, Foam

Solubility in Water: Yes 		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: N/A

Physical Description: Milky white emulsion with slight ammonia cal odor.          	| Unusual Fire & Explosion Hazards: None

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|        	 SECTION 6 - STORAGE AND HANDLING INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid): Avoid metals and other chemicals   		| 	over three high.

Hazardous Decomposition Products: Do not use on aluminum or soft metals. 		|.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Eye or ingestion					|

Skin: Redness and drying                                       			                 | Skin:  Wash  thoroughly with water. If irritation persists, call physician

|

Eyes: Discomfort, redness, and moderate irritation			                 | Eyes: Flush with water for at least 15 minutes. Call physician if irritation continues.

|

Inhalation: N/A                                                                                    		| Inhalation: None

|

Ingestion: Slight mucosal damage along with stomach cramps, nausea and/or                | Ingestion: Immediately drink large quantities of water. Contact a physician

Vomiting.                                       		                 |

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: None required                                                   		| Ventilation: Local exhaust

|

Protective Gloves: None                                                          			| Eye Protection: Goggles or face shield.

Other Protective Equipment: None required				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN.

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Wear protect- 		| Waste Disposal Method: At appropriate facility in accordance with applicable

ive clothing. Sprinkle hydrated lime or soda ash. Scrape up and remove.  		| laws and regulations.

Small - dilute with water and mop up. Wear gloves and goggles always.    		|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------