N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                      	           Effective Date: August 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 				 Revised: Sept. 2004

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: CLEAN-IT-ALL 		Product ID#: 105550			HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: All-purpose cleaner					HEALTH        |2|   	            SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|           SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0| 	     MODERATE 2 MODERATE  	TOXICITY    |2|

Virginia Beach, VA 23452                                                                                		              SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number:  (800) 255-3924						     	          MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Dimethyl Benzyl Ammonium Chlorides               	68391-01-5          	0.8                           	N/A                 	N/A                          	No

Dimethyl Ethylbenzyl Ammonium Chlorides          	68956-79-6          	0.8                           	N/A                 	N/A                          	No

Sodium Metasilicate                               		6834-92-0          	2.5                           	N/A                 	N/A                          	No

EDTA                                                		64-02-8          	1.0                           	N/A                 	N/A                          	No

Anionic Surfactant                               		26027-38-3          	5.2                           	N/A                 	N/A                          	No

Tetrapotassium Pyrophosphate                      	7320-34-5          	5.2                           	N/A                 	N/A                          	No

Water                                             		7732-18-5         	82.5                           	N/A                 	N/A                          	No

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              	|                             SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212o       		Specific Gravity: 1.07                     	| Flash Point (Method Used): 200°

Vapor Pressure: 20mm Hg@68F   	Vapor Density:   (Air=1): Heavier than air 	| Upper Explosive Limit: N/A

% Volatile: 85                		pH: 13.1  				| Extinguishing Media: CO2, Dry Chemical, Synthetic foam or Water.

Solubility in Water: Complete						|	Type BC or ABC

Evaporation Rate:(Water=1): Slower than ethyl ether         			| Special Fire Fighting Procedures: Self-contained breathing apparatus.

Physical Description: Green liquid, mint scent         		            		| Unusual Fire & Explosion Hazards: Dust may be explosive. Hydrogen can be formed

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------                  	        SECTION 5 - REACTIVE DATA                             		|                       SECTION 6 - STORAGE AND HANDLING INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Avoid overheating or freezing.

Incompatibility (Materials to Avoid): Strong Oxidizing Agents            		| Store in a cool, well-ventilated area. Avoid sparks or flames. Ground

Hazardous Decomposition Products: Nitrous Oxides and Ammoniacal vapors   	| 	equipment when transferring or using material.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:                                                  				|

Skin: Prolonged or repeated contact may cause irritation              			| Skin: Wash skin and remove contaminated clothing. Do not reuse clothes.

Eyes: Irritation                                                      				| Eyes: Flush eyes immediately with plenty of water for at least 15 minutes.

Inhalation: Irritation, chemical burns, vapors may cause drowsiness,  		| Inhalation: Remove to fresh air. Use oxygen or artificial respiration, if

nausea, loss of motor skills, or disorientation                     			|             	needed.

Ingestion: Chemical burns                                             			| Ingestion: Consult physician immediately.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: If vapors, fumes or dusts are present use MSHA or		| Ventilation: provide adequate ventilation

NIOSH approved respirator, a fresh air breathing apparatus, or a SCUBA.		|

Protective Gloves: impervious                                            			| Eye Protection: Goggles with full face shield

Other Protective Equipment: equipment to prevent prolonged or repeated contact	|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Ventilate.    		| Waste Disposal Method: Dispose in accordance with all Federal, State, and Local

Small spills - soak with absorbent, shovel waste into containers, wash   		| 	regulations.

with water. Large spills - recover material for reprocessing and disposal.      		|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------