N/A = Not Applicable                                          		  MATERIAL SAFETY DATA SHEET                              		Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 			             Revised Date: MARCH 2008

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: D-STAINED LIQUID      		Product ID#: 113884  	HMIS SYMBOL          	HMIS   NFPA     		NFPA SYMBOL

Generic Name: Destainer 						HEALTH  |2|                          SEVERE 4 EXTREME  		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC				FLAMMABILITY  |0|           SERIOUS 3 HIGH		REACTIVITY  |1|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |1| 	     MODERATE 2 MODERATE   	TOXICITY    |1|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT     		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924			             			   	          MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                  	TWA-TLV             	STEL-TLV          	CARCINOGEN

MG/M3               	MG/M3

Lithium Hypochlorite                              		13840-33-0       	0 - 10                    	  N/A                 	N/A                          	No

Water                                             		7732-18-5        	to 100                           N/A                 	N/A                          	No

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                     	|                       SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212oF     		Specific Gravity: 1.05                     	| Flash Point (Method Used): N/A

Vapor Pressure: 20mm Hg @68F  	Vapor Density:      (Air=1): > 1           	| Upper Explosive Limit: N/A

% Volatile: 98                		pH: 11.0                                   		| Extinguishing Media: N/A

Solubility in Water: Yes      		Evaporation Rate:   (Water=1): < 1         	| Special Fire Fighting Procedures: None

Physical Description: Pale greenish in color with a sweet odor.          		| Unusual Fire & Explosion Hazards: None

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                 SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid): Acids and ammonia                  		| over three high.

Hazardous Decomposition Products: Chloramines                           		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: According to DOT, skin testing on rabbits with a 9% and 12% sol.		| Skin: Wash with plenty of water for at least 15 minutes. Call physician if

showed erythema and adema, but no signs of corrosivity.                      	|          irritation persists.

Eyes: Severe irritation or burns.                                     			| Eyes: Immediately flush with plenty of water for at least 15 minutes. Seek

|           medical attention.

Inhalation: Irritation to mucous membranes. If mixed with an acid,    		| Inhalation: Remove to fresh air. If irritation persists, seek medical aid.

chlorine gas is released (extremely irritating to respiratory tract).		|

Ingestion: Irritation of mouth, throat and stomach with pain and      		| Ingestion: DO NOT induce vomiting. If conscious, give large quantities of milk

possible ulceration. Esophageal strictures occur rarely.                     	|                  or water. Call a physician.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Not required.                                    			| Ventilation: Local exhaust ventilation recommended where misting occurs.

Protective Gloves: Rubber                                                			| Eye Protection: Chemical splash goggles or face shield.

Other Protective Equipment: Rubber apron where concentrated product may be splashed.

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Rinse area    		| Waste Disposal Method: Dilute with copious amounts of cold water.

with copious amounts of water.

DOT INFORMATION

LITHIUM HYPOCHLORITE, DRY, 5.1, UN1471, PGII|