N/A= Not Applicable			MATERIAL SAFETY DATA SHEET		Effective Date: Oct 2002

# SECTION 1- PRODUCT IDENTIFICATION                                              Revised Feb. 2004

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: DEEP FAT FRYER CLEANER   Product ID#: 112350     HMIS SYMBOL                   HMIS     NFPA               NFPA SYMBOL

Generic Name: Fryer Cleaner				           HEALTH              4          SEVERE 4 EXTREME      FIRE                0

Supplier’s Name: BIRSCH INDUSTRIES			           FLAMMIBILITY 0        SERIOUS 3 HIGH	       REACTIVITY 3

Supplier’s Address: 476 Viking Drive, Suite 102		           REACTIVITY      3   MODERATE 2 MODERATE   TOXICITY      4

Virginia Beach, VA  23452		                                                      SLIGHT 1 SLIGHT           SPECIAL         3

Emergency Phone Number:  (800) 255-3924					              MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2- INGREDIENTS

-----------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME		CAS#		WT.		TWA-TLV	STEL-TLV	CARCINOGEN

MG/M3		MG/M3

*Sodium Hydroxide		1310-73-2		< 75.0		2C		N/A		NO

Sodium Metasilicate	                10213-79-3		< 10.0		N/A		N/A		NO

Soda Ash			  497-19-8		<25.0		N/A		N/A		NO

*This product contains toxic chemicals subject to the reporting

requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 373.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3-PHYSICAL DATA				SECTION 4- FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): N/A	Specific Gravity: N/A			Flash Point (Method Used): N/A

Vapor Pressure: N/A 		Vapor Density:	 (Air=1): N/A	Upper Exposure Limit: N/A

% Volatile: N/A		pH: 14				Extinguisher Media: N/A

Solubility in Water: Yes	Evaporation Rate:	(Water=1):N/A	Special Fire Fighting Procedures: N/A

Physical Description: White powder				Unusual Fire & Explosion Hazards: reacts with amphoteric metals, 							makes H2.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5-REACTIVE DATA				SECTION 6- STORAGE AND HANDLING INFORMATION

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable	Hazardous Polymerization: May occur 		Precautions to be Taken in Handling and Storage: Do not stack

Incompatibility (Materials to Avoid): Do not mix with ANY other 		cases or pails over two high.

Chemicals

Hazardous Decomposition Products: DO NOT MIX WITH ALUMINUM.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7- HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure:					First Aid Procedures:

Primary Route of Entry:

Skin: Severe skin irritant.  May cause reddening, swelling, and 		Skin: Flush with water. Seek medical attention if irritation persists.

possible skin damage.					Eyes: Flush with large quantities of water, holding eyelids open.

Eyes: Severe eye irritant.  Liquid and mist may damage eyes, causing	Seek medical attention if irritation persists.

corneal damage.						Inhalation: Get to fresh air.  Seek medical attention if irritation

Ingestion: Irritating and corrosive to the mouth and throat.  May cause 	persists.

headache, nausea, abdominal pain, vomiting and diarrhea.	Ingestion: Do not induce vomiting.  Drink large quantities of water.  Seek medical attention immediately.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8-SPECIAL PROTECTION INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: use in well ventilated area.	Ventilation: Provide local exhaust to keel TLV of Section 2

Protective Gloves: Waterproof	ingredients below acceptable limit.

Other Protective Equipment : Rubber aprons and shoes.	Eye Protection: Safety Glasses.

PLEASE READ THE DIRECTIONS ON THE PRODUCT LABEL.  THEY ARE YOU BEST GUIDE FOR THIS PRODUT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9- SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Dike & absorb	Waste Disposal Method: In accordance with all Federal, State and

Spill with inert material such as savdust.  Transfer to container for disposal.	Local regulations.