N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                       		Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION			             Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: FLEET  LIQUID TRUCK/CAR WASH        Product ID#: 122016          	HMIS SYMBOL     		   HMIS   NFPA      		NFPA SYMBOL

Generic Name: Vehicle Liquid Wash					HEALTH        |1|   	              SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|             SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0| 	       MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452						               SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						                             MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Sodium Dodecylbenzenesulfonic Acid               	27176-87-0         	<10.0                          N/A                 	N/A                          	No

Tetra-sodium Pyrophosphate                        		7320-34-5         	< 5.0                          	N/A                 	N/A                          	No

Nonionic Surfactant                               		9016-45-9         	< 5.0                          	N/A                 	N/A                          	No

Coconut Diethanolamine                            		61789-19-3        	< 5.0                          	N/A                 	N/A                          	No

Tetra sodium EDTA                                  		64-02-8           	< 3.0

Water                                             		7732-18-5        	to 100                       	N/A                 	N/A                          	No

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                            SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212oF       		Specific Gravity: 1.2                      	| Flash Point (Method Used): None

Vapor Pressure: 20 mm Hg @68oF 	Vapor Density:      (Air=1) : >1            	| Upper Explosive Limit: N/A

% Volatile: 85                		pH:  9 - 9.5                                    	| Extinguishing Media: Water foam, CO2, and dry chemicals

Solubility in Water: Yes      		Evaporation Rate:   (Water=1) : <1          	| Special Fire Fighting Procedures: Water & regular foam cause excessive frothing

Physical Description: Red liquid with citrus odor.                       			| Unusual Fire & Explosion Hazards: None

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	    SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid): None                               			| 	over three high.

Hazardous Decomposition Products: None                                   			|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: May produce irritation or corrosive burns to skin. Dilute       		| Skin: Immediately wash with soap and plenty of water while removing contamin-

solutions may cause dermatitis after prolonged exposure.                		|	ated clothing. Seek medical aid.

Eyes: May cause irritation or burns.                                  			| Eyes: Immediately flush eyes with plenty of water for at least 15 minutes. Seek

|	medical aid.

Inhalation: Mists may produce respiratory irritation.                 			| Inhalation: Remove to fresh air.

Ingestion: May cause irritation and corrosive burns to mouth throat   		| Ingestion: DO NOT induce vomiting. Give large quantities of water if conscious.

and stomach.                                                            			|	Seek medical aid.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Where established exposure limits may be exceeded		| Ventilation: Local exhaust is recommended

use a NIOSH-approved container for disposal.			|

Protective Gloves: Rubber                                                			| Eye Protection: Chemical splash goggles.

Other Protective Equipment: Rubber apron is recommended.			|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Prevent eye   		| Waste Disposal Method: Burn in approved incinerator or bury away from water

and skin contact.                                                        				| supplies. Observe Local, State and Federal regulations.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------