N/A = Not Applicable                                      	                   MATERIAL SAFETY DATA SHEET                                       		     Effective Date: Oct 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION			            Revised Date: Nov 2008

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: FREEZER CLEANER               Product ID#: 165000		HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Freezer Cleaner 						HEALTH        |2|   	            SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|           SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0| 	     MODERATE 2 MODERATE  	TOXICITY    |2|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						                            MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

Propylene Glycol				57-55-60		45 to 50		                                                                    	No

Potassium Hydroxide				1310-58-3		0 to 3		Cmg/m3		N/A		No

Nonionic Surfactant				9016-45-9		0 to 5

IPA					67-63-0		0 to 5

Water                                             		7732-18-5       	to 100                           N/A                 	N/A                          	No

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                            SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >275 F    		Specific Gravity: .96                    	| Flash Point (Method Used): N/A

Vapor Pressure: 20 mm Hg @68F 	Vapor Density:      (Air=1): >1            	| Upper Explosive Limit: None

% Volatile: 50+                		pH: 11.5 to 12.0                                    	| Extinguishing Media: N/A

Solubility in Water: Complete 		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: N/A

Physical Description: Slightly viscous clear liquid with characteristic odor.           	| Unusual Fire & Explosion Hazards: N/A

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	    SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Avoid contact with eyes, skin and

|    clothing.           .

Incompatibility (Materials to Avoid): Strong oxidizers.				|  Keep out of reach of children. For use by trained personnel only.

Hazardous Decomposition Products: Alkaline vapors in a fire.                                 	|  Keep container closed during storage. For institutional and industrial use only.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Skin irritant. May cause moderate reddening, and swelling. 		| Skin: Flush with water. Seek medical attention if irritation persists.

|

Eyes: Severe eye irritant. Liquid and mist may damage eyes, causing   		| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

corneal damage.                                                 				|       	attention if irritation persists.

Inhalation: Vapors and mist may be irritating to mucous membranes in  		| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

The nose, throat, and lungs.                                          			|

Ingestion: Irritating and corrosive to the mouth and throat. May      		| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

cause headache, nausea, abdominal pain, vomiting and diarrhea.		|       	medical attention immediately.

Excessive amounts may cause collapse.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilation area.                    			| Ventilation: Provide local exhaust to keel TLV of Section 2 ingredients below

|       	acceptable limit.

Protective Gloves: Water Proof                                            			| Eye Protection: Safety glasses.

Other Protective Equipment: N/A					|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Mop up or otherwise absorb    | Waste Disposal Method: In accordance with all applicable Federal, State and

And hold for disposal.						| Local regulations.

|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------