N/A = Not Applicable                                     MATERIAL SAFETY DATA SHEET                                  Effective Date: Oct 2002

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1- PRODUCT IDENTIFICATION		                   Revised Date: Feb 2004

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: GRAFFITTI REMOVER GEL      	PRODUCT ID #: 1280804     HMIS SYMBOL                         HMIS   NFPA                NFPA SYMBOL

Generic Name: CITRUS BASED GRAFFITTI REMOVER 		            HEALTH        |1|                    SEVERE 4 EXTREME      FIRE        |2|

Supplier's Name: BIRSCH INDUSTRIES, INC.			            FLAMMABILITY  |2|          SERIOUS 3 HIGH               REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102		            	            REACTIVITY    |0|         MODERATE 2 MODERATE  TOXICITY    |1|

Virginia Beach, VA  23452					           	       SLIGHT 1 SLIGHT          SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                                                 MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

SAFE HANDLING PROCEDURES: Precautions To Be Taken In Handling and Storage: Usual precautions for combustible liquids. Keep temperatures below 140oF. Store in tightly sealed, full containers. Clean up all spills. All handling equipment should be grounded. Product may expand slightly in storage causing

pressure to build on container. Open container carefully if product appears to be under pressure. Wash thoroughly after handling.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        	TWA-TLV             	STEL-TLV          	CARCINOGEN Citrus Terpenes                                   		5989-27-5         	<55                         	N/A                 	N/A                          	No

Nonionic Surfactant                              		9016-45-9          	<10                                                                          		No

Coconut Diethanolamide			8051-30-7		<10						No

This product contains no hazardous compounds as defined by OSHA Regulations (29 CFR 19 10). Product is a by-product of citrus, entirely of natural origin,

and believed to contain no artificial flavors, sulfites, nitrites or pesticides residue exceeding tolerances established by the FDA.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                         	|                             SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 310oF            		Specific Gravity: 0.972               	| Flash Point (Method Used): >200OF (TCC)

Vapor Pressure:<10mm Hg @68F        	Vapor Density: (Air=1) >1      	| Upper Explosive Limit: N/A

% Volatile: 90+                     		pH: 11.5 to 12.0                     	| Extinguishing Media: CO2, Dry Chemical, Foam.

Solubility in Water: Emulsifies     	Evaporation Rate:(Ether=1) <1   	| Special Fire Fighting Procedures: Do not use water.

Physical Description: Thin orange/brown liquid with pleasant odor.              	| Unusual Fire & Explosion Hazards: Drums heated by fire can explode.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                       SECTION 6 - STORAGE AND HANDLING INFORMATION

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable

Hazardous Polymerization: None                				| Avoid contact with eyes, skin and clothing. For use by trained personnel only.

Incompatibility (Materials to Avoid): None Known            		| Keep container closed during storage. Keep out of reach of children.

Hazardous Decomposition Products: Carbon monoxide possible if burned.    	| For institutional and industrial use only.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  		| First Aid Procedures

Primary Route of Entry:					|

Skin: Defatting to skin with prolonged or repeated use.               		| Skin: Wash thoroughly with water. If irritated persists, seek medical attention.

Eyes: Eye irritant. May cause redness, burning, or tearing.           		| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

|           attention if irritation persists.

Inhalation: Vapors and mists may be irritating to mucous membranes    	| Inhalation: Remove to fresh air. Seek medical attention if irritation persists.

in the nose, throat, and lungs.

Ingestion: May be irritating to the mouth, throat, and                		| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

gastrointestinal system.                                        			 medical attention immediately.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilated area.                     		| Ventilation: Provide local exhaust to keep TLV of Section 2 ingredients

|        	    below acceptable limit.

Protective Gloves: Solvent proof.                                        		| Eye Protection: Safety glasses.

Other Protective Equipment: N/A        				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------                                                       SECTION 9 - SPILL OR LEAK PROCEDURES

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Wipe up with  	| Waste Disposal Method: Small spills - flush into sewer system. Large - contain

damp mop, sponge or rags and discard. Flush area with clean water if     	| with absorbent material and discharge in accordance with all Federal, State and

necessary. Avoid discharge into open waterways.                          		| Local regulations.

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------