N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                               Effective Date:

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION	              Revision:        Revision Date:

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: Industrial Spray Foam		                Product ID#: 140400			HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Cleaner/Degreaser/Floor Stripper			HEALTH        | 3  |   	            SEVERE 4 EXTREME   		FIRE        | 0  |

Supplier's Name: BIRSCH INDUSTRIES, INC. 			FLAMMABILITY  | 0  |          SERIOUS 3 HIGH      		REACTIVITY  | 0  |

Supplier's Address: 476 Viking Drive, Suite 102 			REACTIVITY    | 0  | 	     MODERATE 2 MODERATE  	TOXICITY    | 3  |

Virginia Beach, VA  23452	  		                 	 SLIGHT 1 SLIGHT    		SPECIAL     | 0  |

Emergency Phone Number: (800) 255-3924						         MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

DDBSA				27176-87-0	3-5

Noninic Surfactant			26027-38-3	4-10		N/A		N/A		N/A

Sodium Silicate			1344-09-8		3-5		Not Established          N/A		N/A

*Sodium Hydroxide			1310-73-2		5-8

Versene 100			60-00-4		2-5

Coupling Agent			1300-72-7		<2

Water				7732-18-5              to  100		N/A		N/A		N/A

*This product contains the following toxic chemicals subject to the reporting requirements

Of section 313 of the Emergency Planning and Community Right To Know Act of 1988

And of 40 CFR 372

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                        SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212F	       	Specific Gravity:  1.08		| Flash Point (Method Used):  None

Vapor Pressure:20mm Hg@68F		Vapor Density:   (Air=1): <1		| Upper Explosive Limit:  N/A

% Volatile: 80+	              		pH: 10.5				| Extinguishing Media: Water, Fog, CO2, and dry chemicals

Solubility in Water: Yes			                                                     |

Evaporation Rate :( Water=1): <1					| Special Fire Fighting Procedures:. Water & regular foam may cause frothing

Physical Description:	 Clear liquid with bland odor				| Unusual Fire & Explosion Hazards: None

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------                  	        SECTION 5 - REACTIVE DATA                             		|                       SECTION 6 - STORAGE AND HANDLING INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------Stability:          	Hazardous Polymerization: Will not occur			| Precautions to be Taken in Handling and Storage: Keep out of reach of children

Incompatibility (Materials to Avoid): Do not mix with other chemicals                              | For use by trained personnel only. Keep container closed during storage. For

Hazardous Decomposition Products: Alkaline vapors in a fire		                  | institutional and industrial use only. WARNING CORROSIVE

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:                                                  				|

Skin: Severe skin irritant. May cause redding, swelling & possible skin damage	| Skin: Flush with water; seek medical attention if irritation persists.

Eyes: Liquid, & may damage eyes, causing corneal damage	                                   | Eyes: Remove contacts flush immediately with large quantities of water for 15

Inhalation: Vapors and mist may be irritating to mucous membranes in the		|     mins.  holding eyelids open. Seek medicals attention if irritation persists.

throat and lungs					|Inhalation: Get to fresh air seek medical attention if irritation persist

Ingestion: Irritating and corrosive to the mouth and throat. May cause headache,	| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

Nausea, abdominal pain, vomiting and diarrhea.                                                                         medical attention immediately.

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilated areas.				| Ventilation: Provide local exhaust to keep TLV of ingredients

|                         below limits.

Protective Gloves: Waterproof		                                            		| Eye Protection:  Safety glasses

Other Protective Equipment: N/A					|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Dike and absorb spills	   Waste Disposal Method: Dispose in accordance with all Federal, State, and

with inert material and transfer to container for disposal.                                                         local  regulations.							                                                                                                                            |

|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------