N/A = Not Applicable                                                                       MATERIAL SAFETY DATA SHEET                                       		Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION			           Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: LOD-NEUTRAL 32	Product ID#: 145499			HMIS SYMBOL     		HMIS     NFPA      		NFPA SYMBOL

Generic Name: Disinfectant and Cleaner					HEALTH        |1|   	             SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|            SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0| 	      MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452                                                                                                                SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						                             MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Alkyl dimethyl benzyl ammonium chloride	               		68424-85-1	1.08		N/A                 	N/A                          	No

Di (octyl-decyl) dimethyl ammonium chloride		68424-95-3	1.63		N/A                 	N/A                          	No

Ethyl alcohol					64-17-5		1.50		N/A                 	N/A                         	No

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              	|                        SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212o F      		Specific Gravity: 1.02                     	| Flash Point (Method Used): 200oF

Vapor Pressure: 20mm Hg@68oF   	Vapor Density:   (Air=1): Heavier than air 	| Upper Explosive Limit: N/A

% Volatile: 81                		pH: 7.5-8.0                                   	| Extinguishing Media: CO2, Dry Chemical, Synthetic foam or Water. Type BC or ABC

Solubility in Water: Complete 		Evaporation Rate:(Water=1): Slower than    	| Special Fire Fighting Procedures: Self-contained breathing apparatus.

Physical Description: Yellow liquid, lemon odor.      				| Unusual Fire & Explosion Hazards: Dust may be explosive. Hydrogen can be formed

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                     SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Avoid overheating or freezing.

Incompatibility (Materials to Avoid): Strong Oxidizing Agents            		| Store in a cool, well-ventilated area. Avoid sparks or flames. Ground

Hazardous Decomposition Products: Nitrous Oxides and Ammoniacal vapors   	| 	equipment when transferring or using material.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:                                                  				|

Skin: Prolonged or repeated contact may cause irritation              			| Skin: Wash skin and remove contaminated clothing. Do not reuse clothes.

Eyes: Irritation                                                      				| Eyes: Flush eyes immediately with plenty of water for at least 15 minutes.

Inhalation: Irritation, chemical burns, vapors may cause drowsiness,  		| Inhalation: Remove to fresh air. Use oxygen or artificial respiration, if

nausea, loss of motor skills, or disorientation                     			|             	needed.

Ingestion: Chemical burns                                             			| Ingestion: Consult physician immediately.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: If vapors, fumes or dusts are present use MSHA or		| Ventilation: provide adequate ventilation

NIOSH approved respirator, a fresh air breathing apparatus, or a SCUBA.		|

Protective Gloves: impervious                                            			| Eye Protection: Safety Glasses

Other Protective Equipment: equipment to prevent prolonged or repeated contact	|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Ventilate.   	| Waste Disposal Method: Dispose in accordance with all Federal, State, and Local

Small spills - soak with absorbent, shovel waste into containers, wash   		| 	regulations.

with water. Large - recover material for reprocessing and disposal.      		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------