N/A = Not Applicable                                      		 MATERIAL SAFETY DATA SHEET                                       		Revised Date: Oct 2005

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION				Effective Date: Oct 2002

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: LEMON FURNITURE POLISH            Product ID#: 172846		HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Furniture Polish                                          			HEALTH        |1|                     SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|            SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 3412 Strathmore Avenue 				REACTIVITY    |0| 	      MODERATE 2 MODERATE  	TOXICITY    |1|

Norfolk, VA 23504                                                                                		              SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                             	           MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                                  CAS #             	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Polydimethylsiloxane/water emulsion                           N/A               	15 - 22                         N/A                 	N/A                          	No

Dowanol EB                                             	            111-76-2        	                   0-3                             N/A                 	N/A                          	No

Lemon Scent                                          	              N/A           	                 2 -  4                           	606                 	909                          	No

Nonylphenol Polyethylene                                        26027-38-3                       trace                           	N/A                 	N/A                          	No

Water                                             	           7732-18-5        	to 100                         	N/A                 	N/A                          	No

This product contains no toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                            SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212o      		Specific Gravity: .96                      	| Flash Point (Method Used): None to boil

Vapor Pressure: N/A           		Vapor Density:      (Air=1): N/A           	| Upper Explosive Limit: None

% Volatile: N/A               		pH: 7.0-7.5                                		| Extinguishing Media: N/A

Solubility in Water: Yes      		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: None

Physical Description: Red emulsion liquid, slight lemon odor             		| Unusual Fire & Explosion Hazards: None

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|                       SECTION 6 - STORAGE AND HANDLING INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      			| Precautions to be Taken in Handling and Storage: Do not stack boxes over 5 high.

Incompatibility (Materials to Avoid): Do not mix with ANY other chemicals		|

Hazardous Decomposition Products: None                                   			|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: May be irritating to skin. Solutions may cause dermatitis after 		| Skin: Immediately wash with soap and plenty of water while removing

prolonged exposure.                                             			|          contaminated clothing.

Eyes: Can cause irritation or burns.                                  			| Eyes: Immediately flush eyes with plenty of water for at least 15 minutes. Seek

|            medical aid.

Inhalation: Mists may produce respiratory irritation.                 			| Inhalation: Remove to fresh air Seek medical aid if irritation persists.

Ingestion: Can cause irritation and corrosive burns to mouth throat   		| Ingestion: DO NOT induce vomiting. Give large quantities of water if conscious.

and stomach.                                                            			|	 Seek medical aid.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Where established exposure limits may be exceeded		| Ventilation: Local exhaust is recommended

use a NIOSH-approved container for disposal.                         	|

Protective Gloves: Rubber                                                			| Eye Protection: Chemical splash goggles.

Other Protective Equipment: None                				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN  THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be taken in Case Material is Released or Spilled: Mop, vacuum or		| Waste Disposal Method: Dilute with plenty of water and flush into sewer system.

wash away with water.					|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------