N/A= Not Applicable			MATERIAL SAFETY DATA SHEET		EFFECTIVE DATE:  Oct 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1-PRODUCT IDENTIFICATION		  Revised Date: Feb. 2004

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name:  LOW TEMP SANITIZER	Product ID#:  146785	  HMIS SYMBOL	         HMIS  NFPA	       NFPA  SYMBOL

Generic Name:  Bleach				  HEALTH  4                SEVERE 4 EXTREME   FIRE  0

Supplier’s Name:  Birsch Industries, Inc.			  FLAMMIBILITY 0  SERIOUS 3 HIGH	       REACTIVITY  4

Supplier’s Address:  476 Viking Drive, Suite 102		  REACTIVITY 4 MODERATE 2 MODERATE TOXICITY 4

Virginia Beach, VA  23452					  SLIGHT 1 SLIGHT         SPECIAL 1

Emergency Phone Number: (800) 255-3924				                MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2- INGREDIENTS

------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME:		CAS#		WT %	TWA-TLV	STEL-TLV	CARCINOGEN

MG/M3		MG/M3

*Sodium Hypochlorite	7681-52-9		<10.0	N/A		N/A		NO

Water			7732-18-5		to 100	N/A		N/A		NO

* This product contains toxic chemicals subject to the reporting

requirements of Section 313 of the Emergency Planning and

Community Right-To-Know Act of 1988 and of 40 CFR 372.

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3- PHYSICAL DATA			SECTION 4- FIRE AND EXPLOSION HAZARD DATA

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212oF	Specific Gravity:  1.14-1.165		Flash Point (Method Used):  N/A

Vapor Pressure: Like water	Vapor Density:  (Air=1): Like water	Upper Explosive Limit:  N/A

% Volatile:  <10		pH: 12.7				Extinguishing Media: Product is not flammable

Solubility in Water: 	            Complete Evaporation Rate: (Water = 1): N/A	Special Fire Fighting Procedures:  Use self- contained

Physical Description:  Clear pale yellow liquid, chlorine odor		breathing apparatus

Unusual Fire & Explosion Hazards:  None Known

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5-REACTIVE DATA			SECTION 6- STORAGE AND HANDLING INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Unstable	Hazardous Polymerization: Will not occur		Precautions to be Taken in Handling and Storage:

Incompatibility (Materials to Avoid) : Mixtures with acids or ammonia	Keep container closed.  Wash thoroughly after handling

Hazardous Decomposition Products:  Chlorine Gas

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7- HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure					First Aid Procedures

Primary Route of Entry: Skin

Skin:  According to DOT, skin testing on rabbits with a 9% and 12% sol.	Skin:  Wash with plenty of water for at least 15 minutes

showed erythema and adema, but no signs of corrosivity.		Call a physician if irritation persists.

Eyes:  Severe irritation or burns					Eyes: Immediately flush with plenty of water for at

Inhalation:  Irritation to mucous membranes.  If mixed with an acid, 	least 15 minutes.  Seek medical attention.

chlorine gas is released (extremely irritating to respiratory tract).		Inhalation:  Remove to fresh air.  If irritation persists,

Ingestion:  Irritation of mouth, throat, and stomach with pain and possible	seek medical attention.

ulceration.  Esophageal strictures occur rarely.			Ingestion:  DO NOT induce vomiting.  If conscious,

give large quantities of milk or water.  Call a physician.

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8- SPECIAL PROTECTION INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection:  Not required		Ventilation:  Local exhaust ventilation recommended where misting occurs.

Protective Gloves: Rubber			Eye Protection:  Chemical splash goggles or face shield.

Other Protective Equipment:  Rubber apron where

concentrated products may be splashed.

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL.  THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9- SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled:  Neutralize	Waste Disposal Method:  In accordance with all

With soda ash and water.  Transfer slurry to container for disposal.		Federal, State and Local regulations.