N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                       		Effective Date: Oct 2002

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 				 Revised Date: Feb. 2004

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: MPC PLUS NEUTRAL CLEANER	Product ID#: 151751 		HMIS SYMBOL     		   HMIS   NFPA      		NFPA SYMBOL

Generic Name: Cleaner                                                        			HEALTH        |1|   	              SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|             SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0| 	       MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452						               SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                              	           MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                  	TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Sodium Lauryl Sulfate			151-21-3		<5

Sodium Dodecylbenzinesulfonic Acid               	27176-87-0         	<15.0                          N/A                 	N/A                          	No

Nonionic Detergent                               		26027-38-3         	< 5.0                          	N/A                 	N/A                          	No

*Ethylene Glycol Monobutyl Ether                    	111-76-2         	< 5.0                           25                 	N/A                          	No

Water                                             		7732-18-5        	to 100                          N/A                 	N/A                          	No

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		    |      SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212o F     		Specific Gravity: 1.02                     	| Flash Point (Method Used): None to boil

Vapor Pressure: N/A           		Vapor Density:      (Air=1): N/A           	| Upper Explosive Limit: None

% Volatile: N/A               		pH: 9.5-10.0                               		| Extinguishing Media: N/A

Solubility in Water: Yes      		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: None

Physical Description: Orange liquid, slight perfume odor                			| Unusual Fire & Explosion Hazards: None

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		            |     SECTION 6 - STORAGE AND HANDLING INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid): Do not mix with ANY other chemicals		| over three high.

Hazardous Decomposition Products: None                                   			|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: May produce irritation or corrosive burns to skin.  Diluted       		| Skin: Immediately wash with soap and plenty of water while removing

solutions may cause dermatitis after prolonged exposure.                		|          contaminated clothing. Seek medical aid.

Eyes: Can cause irritation or burns.                                  			| Eyes: Immediately flush eyes with plenty of water for at least 15 minutes. Seek

|           medical aid.

Inhalation: Mists may produce respiratory irritation.                 			| Inhalation: Remove to fresh air.  Seek medical aid if irritation persists.

Ingestion: Can cause irritation and corrosive burns to mouth throat   		| Ingestion: DO NOT induce vomiting. Give large quantities of water if conscious.

and stomach.                                                            			|	Seek medical aid.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Where established exposure limits may be exceeded		| Ventilation: Local exhaust is recommended.  Use a NIOSH-approved container for Protective Gloves: rubber                                                			|	    disposal.

Other Protective Equipment: None					| Eye Protection: Chemical splash goggles with full-face shield

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL.  THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Mop, vacuum or		| Waste Disposal Method: Dilute with plenty of water and flush into sewer system.

wash away with water.                                                    				|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------