N/A = Not Applicable                                      		  MATERIAL SAFETY DATA SHEET                                   		Effective Date: May 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION 				     Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: MX KNOCK- OUT		Product ID#: 150210          	HMIS SYMBOL                        HMIS     NFPA       		NFPA SYMBOL

Generic Name: Mixing Station Cleaner                                                                                HEALTH        |1|   	           SEVERE 4 EXTREME    		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|          SERIOUS 3 HIGH       		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Ste 102 				REACTIVITY    |0| 	    MODERATE 2 MODERATE   	TOXICITY    |1|

Virginia Beach VA 23452                                                                              		            SLIGHT 1 SLIGHT    		SPECIAL    |0|

Emergency Phone Number:  (800) 255-3924						        	        MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT. %               	TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Sodium Met                                            			6834-92-0          	<5.0                          	N/A                 	N/A                          	No

Sodium Tripoly  Phosphate                            			7758-29-4           	<3.0                          	  5                 	N/A                          	No

Soda Ash                                                   			497-19-8             	<3.0                          	N/A                 	N/A                          	No

Hexylene Glycol                                           			107-41-5          	<2.0                         	N/A                 	N/A                          	No

Versene                                                                                              60-00-4                      <6.0                                                                                                   No

Quarternary Ammonium Compound                                                 64-17-5                       <2.0                            N/A                             N/A                            No

Nonionic Surfactant                                                                          26027-8-3                   <8.0                             N/A                            N/A                             No

Water                                                                                                7732-18-5                  to 100                            N/A                            N/A                             No

This product contains toxic chemicals subject to the reporting requirements of  Section 313 of  the Emergency Planning and Community Right-To-Know Act

Of 1988 and of  40 CFR  372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------	           		SECTION 3 - PHYSICAL DATA                              		|                             SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F):>212oF      		Specific Gravity: 1.06                      	| Flash Point (Method Used): N/A

Vapor Pressure: 20mmHg@68F.   	Vapor Density:      (Air=1): >1            	| Upper Explosive Limit: N/A

% Volatile: 85                		pH: 11.5-12.0                                	| Extinguishing Media: Water, CO2, foam and chemical

Solubility in Water: Yes      		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: None

Physical Description: Green watery thin liquid with a bland odor                   		| Unusual Fire & Explosion Hazards: None

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	    SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be taken in Handling and Storage: Keep out of reach of children.

Incompatibility (Materials to Avoid): Do not mix with other chemicals.   		| For use by trained personnel only. Keep container closed during storage. For

Hazardous Decomposition Products: N/A                                    			| institutional and industrial use only.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures:

Primary Route of Entry:						|

Skin: Prolonged or repeated contact may cause dermatitis	           	                 [Skin:  Flush with. Seek medical attention if irritation persists.

Eyes: Eye irritant. May cause redness or swelling.                    			|

Inhalation: Vapors and mist may be irritating to mucous membranes                               |Eyes:  Flush with large quantities of water, holding eyelids open. Seek medical attention

In the throat, and lungs                                                                                              | if irritation persists.

Ingestion: May be irritating to mouth, throat, May cause vomiting and diarrhea              | Inhalation:  Get to fresh air. Seek medical attention of irritation persists.

Headache, nausea and abdominal pain.		|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilated area                                   		| Ventilation: Local exhaust to keep TLV of Section 2 ingredients below

|	acceptable level.

Protective Gloves: Water proof                                              			| Eye Protection: N/A

Other Protective Equipment: None required      				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be taken in Case Material is Released or Spilled: Dike and absorb with inert 	| Waste Disposal Method: In accordance with Federal, State, and Local

material such as vermiculite. Transfer to containers for disposal.			| 	regulations.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------