N/A = Not Applicable               			                      MATERIAL SAFETY DATA SHEET    			Effective Date: Oct 2002

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION			       Revised Date: July 2004

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: NEW KEM-TRONICS	Product ID#  152880                            	HMIS SYMBOL          	HMIS   NFPA       		NFPA SYMBOL

Generic Name: Cleaner/Degreaser/Floor Stripper 				HEALTH        |3|                  SEVERE 4 EXTREME    		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|        SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0|       MODERATE 2 MODERATE   	TOXICITY    |3|

Virginia Beach, VA  23452					 	           SLIGHT 1 SLIGHT     		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						  	       MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                               	CAS #           	WT. %               	TWA-TLV      	STEL-TLV           	CARCINOGEN

MG/M3               	MG/M3

*Monoethanolamine                                  	141-43-5           	5 - 8                           	3                  	6                            	No

Nonionic Surfactant                             	26027-38-3           	4 - 6 		N/A		N/A                      	No

Sodium Silicate			1344-09-8		5 - 10		Not Established			N0

2 Butoxy Ehanol			111-76-2		12 - 20

*Sodium Hydroxide			1310-73-2		5 - 8

Water                                            		7732-18-5          	to 100                         N/A                	 N/A                          	No

*This product contains the following toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|    SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212oF      Specific Gravity: 1.1                     			| Flash Point (Method Used): None

Vapor Pressure: 20 mm Hg@68oF  Vapor Density:      (Air=1): <1            		| Upper Explosive Limit: N/A

% Volatile: 80+               pH: 13                                     				| Extinguishing Media: Water fog, CO2, and dry chemicals

Solubility in Water: Yes      Evaporation Rate:   (Water=1): <1          		| Special Fire Fighting Procedures: Water and regular foam may cause frothing

Physical Description: Thin, purple liquid with mild scent                            			| Unusual Fire & Explosion Hazards: None

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		        |    SECTION 6 - STORAGE AND HANDLING INFORMATION

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur     		| Precautions to be Taken in Handling and Storage: Keep out of reach of children.

Incompatibility (Materials to Avoid): Do not mix with other chemicals.   		| For use by trained personnel only. Keep container closed during storage. For

Hazardous Decomposition Products: Alkaline vapors in a fire.             		| institutional and industrial use only. WARNING: CORROSIVE

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Severe skin irritant. May cause reddening, swelling, and        		| Skin: Flush with water. Seek medical attention if irritation persists.

possible skin damage.                                            				|

Eyes: Severe eye irritant. Liquid and mist may damage eyes, causing   		| Eyes: Remove contact lenses. Flush immediately with large quantities of water for 15

corneal damage.                                                 				| 	minutes, holding eyelids open. Seek medical attention if irritation persists.

Inhalation: Vapors and mist may be irritating to mucous membranes in  		| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

the throat, and lungs.                                          				|

Ingestion: Irritating and corrosive to the mouth and throat. May      		| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

cause headache, nausea, abdominal pain, vomiting and diarrhea. 		|       medical attention immediately.

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilation area.                    			| Ventilation: Provide local exhaust to keep TLV of ingredients below limits.

Protective Gloves: Waterproof                                            			| Eye Protection: Safety glasses.

Other Protective Equipment: N/A

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST

EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Dike and      		| Waste Disposal Method: Any method in accordance to applicable laws..

absorb spill with inert material and transfer to container for disposal. 		|

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------