N/A = Not Applicable                                      MATERIAL SAFETY DATA SHEET               				Effective Date:  Sept 16, 2005

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION	            		                        Revised Date:

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: PINE FORCE-DET/DIS                               Product  ID#: 1520483         HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Disinfectant                   					HEALTH        |3|   	           SEVERE 4 EXTREME   		FIRE        |1|

Supplier's Name:  BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |1|          SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address:	476 Viking Drive					REACTIVITY    |0|         MODERATE 2 MODERATE  	TOXICITY    |3|

Virginia Beach, VA 23452                                                                                		             SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						                         MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT. %                 	TWA-TLV             	STEL-TLV            	CARCINOGEN

MG/M3               	MG/M3

Dimethyl Benzly Ammonium Chlorides			68391-01-5	0.8		N/A		N/A		No

Dimethyl Ethylbebzyl Ammonium Chlorides		68956-79-6	0.8		N/A		NA		No

Sodium Metasilicate					6834-92-0		2.5		N/A		N/A		No

EDTA						64-02-8		1.0		N/A		N/A		No

Anionic Surfactant					26027-38-3	5.2		N/A		N/A		No

Tetrapotassium Pyrophosphate				7320-34-5		5.2		N/A		N/A		No

Water						7732-18-5		82.5		N/A		N/A		No

*This product contains the following toxic chemicals subject to the reporting requirements of section 313 of the Emergency Planning and Community

Right –To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                             			|                             SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212oF            	Specific Gravity: 1.07 	                 	 | Flash Point (Method Used): >200oF

Vapor Pressure: 20 mm Hg @68 F  	Vapor Density (Air=1): >1 Heavier than air   |  Upper Explosive Limit: N/A

% Volatile: 85                 		pH:13.5 				  | Extinguishing Media: Any.

Solubility in Water: Complete   		Evaporation Rate:(Water=1) >1 Slower than  | Special Fire Fighting Procedures: Self-contained breathing apparatus

Physical Description: Green liquid, Pine odor                                 .      		  | Unusual Fire & Explosion Hazards: Dust may be explosive. Hydrogen can be formed.

VOC = 0

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                          			|                      SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      	         | Precautions to be Taken in Handling and Storage: Avoid overheating or freezing           Incompatibility (Materials to Avoid): Strong oxidizers Agents                                  | Store in a coo, well-ventilated area. Avoid sparks or flames. Ground equipment

Hazardous Decomposition Products: Nitrous Oxides and Ammoniacal Vapors.         |    when transferring or using material.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:   Skin: Prolonged or repeated contact may cause irritation.       |   Skin: Wash skin and remove contaminated clothing. Do not reuse cloths until washed.

Eyes: Irritation may be corrosive.                                                  		                  | Eyes: Flush eyes immediately with plenty of water for at least 15 minutes.                                               I  Inhalation:   Irritation, chemical burns, vapors may cause drowsiness,                             | Inhalation: Get to fresh air. Use oxygen or artificial respirator if needed.

Nausea, loss of motor  skills, or disorientation.                                     		 | Ingestion:  Consult physician immediately. Do not induce vomiting.

Ingestion	: Chemical burns

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: If vapors, fumes or dust are present use MSHA. Or  NIOSH            | Ventilation: Provide adequate ventilation                                          		 approved respirator, a fresh air breathing apparatus or a SCUBA                                          |

Protective Gloves: Impervious                                                              .                                    Eye Protection: Goggles with full face shield.

Other Protective Equipment: Equipment to prevent prolonged or repeated contact.

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Ventilate.		| Waste Disposal Method:  Dispose in accordance with  all Federal, State, and Local

Small spills- soak with absorbent, shovel waste into containers, wash with water.                                Regulations.

Large spills- recover material for reprocessing and disposal.