N/A = Not Applicable                                		                     MATERIAL SAFETY DATA SHEET                                       	Effective Date: Oct 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION			     Revised Date: July 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: PUMP SPRAY DISINFECTANT      	Product ID#: 161540           	HMIS SYMBOL     		 HMIS    NFPA      		NFPA SYMBOL

Generic Name: Disinfectant  						HEALTH        |2|   	              SEVERE 4 EXTREME   	FIRE        |1|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |1|             SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0|            MODERATE 2 MODERATE  	TOXICITY    |2|

Virginia Beach, VA  23452						                SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						            	            MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        	TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Dimethyl benzyl ammonium chlorides               	68391-01-5         	0.105                          		N/A                 	N/A                          	No

Dimethyl ethylbenzyl ammonium chlorides          	68956-79-6         	0.105                          		N/A                 	N/A                          	No

Versene 100                                         		64-02-8         	1.500                          		N/A                 	N/A                          	No

Sodium Metasilicate                               		6834-92-0         	0.250                          		N/A                 	N/A                          	No

*Diethylene glycol monoethyl ether                  	111-90-0         	2.000                          		N/A                 	N/A                          	No

Water                                             		7732-18-5        	to 100                          	N/A                 	N/A                          	No

This product contains *toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                          SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212o F      		Specific Gravity: 0.96                     	| Flash Point (Method Used): >200oF

Vapor Pressure: N.D.          		Vapor Density:      (Air=1): >1            	| Upper Explosive Limit: N.D.

% Volatile: 95                		pH: 11.7                                   		| Extinguishing Media: CO2,Dry Chemical,Synthetic Foam,Water, BC/ABC extinguisher

Solubility in Water: Complete 		Evaporation Rate:   (Water=1): Slower than | Special Fire Fighting Procedures: Self-cont breathing app. Water to cool container

Physical Description: Green, pine scented liquid.				| Unusual Fire & Explosion Hazards: H2, Dusts, and fluorocarbons

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|                       SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Avoid overheating or freezing.

Incompatibility (Materials to Avoid): Strong oxidizing agents, other chemicals	| Store in a cool, well-ventilated area. Avoid sparks and flames. Ground

Hazardous Decomposition Products: Nitrous oxides and ammoniacal vapors   		| 	equipment when transferring or using material.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:                                                  			|

Skin: Prolonged or repeated contact may cause irritation.             			| Skin: Wash off skin and remove contaminated clothing. Do not reuse.

Eyes: Contact causes irritation.                                      			| Eyes: Flush eyes immediately with plenty of water for at least 15 minutes.

Inhalation: Irritation to respiratory pasages, chemical burns,        			| Inhalation: Remove to fresh air. Use oxygen or artificial respiration, if

drowsiness, nausea, loss of motor skills, or disorientation.    			|             	needed.

Ingestion: Chemical burns if corrosive.                               			| Ingestion: If swallowed, consult a physician immediately.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: If vapors, fumes or dusts are present, use NIOSH 		| Ventilation: Provide adequate ventilation.

approved respirator, a fresh air or self-cont. breathing apparatus.			|

Protective Gloves: Impervious.                                           			| Eye Protection: Goggles with full face shield.

|

Other Protective Equipment: to prevent prolonged or repeated contact 		|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Contain.      		| Waste Disposal Method: In accordance with all applicable Federal, State and

Small- use absorbent, shovel waste to containers, flush area with water. 		| 	local regulations.

Large- recover material for reprocessing or disposal.                    			|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------