N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                       		Effective Date: Oct 2002

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION			             Revised Date: July 2004

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: RESTROOM ALL-PURPOSE CLEANER     Product ID#: 165010         	HMIS SYMBOL     		  HMIS   NFPA      		NFPA SYMBOL

Generic Name: Tile Cleaner 						HEALTH        |1|   	              SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name:    BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|             SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0|            MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452					                                 SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number  : (800) 255-3924					                             MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Nonionic Detergent                               		26027-38-3         	<10.0                          N/A                 	N/A                          	No

DPM                                              		34590-94-8         	< 6.0                          	N/A                 	N/A                          	No

Citric Acid                                         		77-92-9         	< 4.0                          	N/A                 	N/A                          	No

*Monoethanolamine                                   		141-43-5         	< 3.0                          	7.5                  	15                          	No

Water                                             		7732-18-5        	to 100                          N/A                 	N/A                          	No

This product contains the following toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                           SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212o       		Specific Gravity: 1.01                     	| Flash Point (Method Used): >200o

Vapor Pressure: 20 mmHg@68oF   	Vapor Density:      (Air=1): N.D.          	| Upper Explosive Limit: Not Flammable

% Volatile: 91                		pH: 6.5                                    		| Extinguishing Media: N/A

Solubility in Water: Complete 		Evaporation Rate:   (Water=1): N.D.        	| Special Fire Fighting Procedures: Not flammable.

Physical Description: Watery thin liquid with a mild citrus odor. 			| Unusual Fire & Explosion Hazards: None known.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             		|        	     SECTION 6 - STORAGE AND HANDLING INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur.     		| Precautions to be Taken in Handling and Storage: Use good personal hygiene

Incompatibility (Materials to Avoid): Metals, oxidizers and nitrosaters  		| 	practice. Wash contaminated clothing and equipment before reuse.

Hazardous Decomposition Products: Normal products of combustion.         		|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Preexisting disorder may be aggravated.                         			| Skin: Flush with water. Call a physician if irritation develops.

|

Eyes: Brief mild irritation. Preexisting disorder may be aggravated.  		| Eyes: Flush with water for at least 15 minutes. Call a physician.

|

Inhalation: Mist may irritate respiratory tract. May aggravate        		| Inhalation: Remove to fresh air.

existing condition.						|

Ingestion: Preexisting disorder may be aggravated.                    			| Ingestion: DO NOT induce vomiting. Drink large quantities of water. Call a

|	physician.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: None required with good ventilation.             		| Ventilation: Maintain good ventilation.

|

Protective Gloves: Rubber or other impervious material.                  		| Eye Protection: Chemical goggles.

|

Other Protective Equipment: None.               				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Use mop or    		| Waste Disposal Method: In accordance with all Federal, State and Local

wet vacuum to collect material for proper disposal. Rinse area with water| regulations.	|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------