N/A = Not Applicable                                      MATERIAL SAFETY DATA SHEET                                       Effective Date: 5/99

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION		               Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: SCUM-B-GONE    	Product ID#: 101590         	HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Non-Acid Soap Scum Remover                   		HEALTH        |2|   	           SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name:  BIRSCH INDUSTRIES, INC. 			FLAMMABILITY  |0|          SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 			REACTIVITY    |0|         MODERATE 2 MODERATE    	TOXICITY    |1|

Virginia Beach, VA  23452					             SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924						         MINIMAL 0  INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT. %             	 PEL		TWA-TLV	STEL-TLV

Water                                    				7732-18-5          	to 100

Tetrasodium EDTA                    				64-02-8		5 to 10

Nonionic Surfactant                                			9016-45-9          	0 to 5

Ispropyl Alcohol                                			67-63-0		0 to 5		400 ppm		400 ppm		500 ppm

Perfume Oil                                            			N/A 		0 to 5

Dye						N/A		0 to 5

*This product contains toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                             SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >210 oF       		Specific Gravity: 1.053                   	| Flash Point (Method Used): N/A

Vapor Pressure: 20 mm Hg @68 F  	Vapor Density:      (Air=1): >1          	| Upper Explosive Limit: N/A      Lower Explosion Limit: N/A

% Volatile: 85+                 		pH: 12.5-13.0                                  	| Extinguishing Media: N/A

Solubility in Water: Complete   		Evaporation Rate:   (Water=1): <1        	| Special Fire Fighting Procedures: Non Flammable

Physical Description: Thin pink liquid with pleasant odor.      			| Unusual Fire & Explosion Hazards: N/A

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                          	   	|                      SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable     Hazardous Polymerization: None   			| Keep out of reach of children. For use by trained personnel only.  Keep container closed during              Incompatibility (Materials to Avoid): Strong acids and oxidizers.		| storage.

| For institutional and industrial use only. Avoid contact with eyes, skin and clothing.  Wear rubber

Hazardous Decomposition Products: Will not occur    			| gloves and protective eyewear when handling. Avoid breathing mist.  Use in well-ventilated area.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:

Skin: Prolonged or repeated contact may cause dermatitis.             		| Skin: Flush with water. Seek medical attention if irritation persists.

Eyes: Eye irritant. May cause redness or swelling.      				| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical attention.

Inhalation: Mist irritating to nose, throat, and lungs, causing choking                	| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

Sensation.

Ingestion: May be irritating to the mouth, throat and gastrointestinal  system.		| Ingestion: Dilute with 2 glasses of water. Seek medical attention if symptoms persist.

Vomiting and diarrhea expected with large doses.

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilated area.                         	| Ventilation: Provide local exhaust to keep TLV of Section 2 ingredients  below

Protective Gloves: Chemical resistant rubber.                               			| acceptable limit.

Other Protective Equipment: Eyewash should be provided near by.                		| Eye Protection: Safety glasses required.

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled:  Floors will become slippery.  Avoid walking in  product.  Keep unessential personnel away.  Mop up or otherwise adsorb and hold disposal.  Avoid discharge to sewer or  open waterways.

Waste Disposal Method: Any method in accordance with local, state and federal laws.  Best method is to recycle or reuse for intended purpose.  Consult local authorities for disposal in public sewer.  Do not dispose of into storm drain, stream, river or ground.  Rinse container thoroughly before discarding in trash.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 10- REGULATORY INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SARA Title III – Section 311/312- Hazard Categories: 	Shipping Information:

No- Fire Hazard					For Containers> 1 Gallon: Cleaning Compound

No- Sudden Release of Pressure Hazard

No- Reactivity Hazard				For 1 Gallon Containers: Cleaning Compound

Yes- Immediate (acute) Health Hazard

No-Delayed (chronic) Health Hazard			For Containers< 1 Gallon : Cleaning Compound

FOR EMERGENCY MEDICAL OR TRANSPORT INFORMATION: CHEM-TEL, Inc. @ 800-255-3924 (24 HOURS A DAY, 7 DAYS PER WEEK)