N/A = Not Applicable                                      		MATERIAL SAFETY DATA SHEET                                               Effective Date: October 2002

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 1 - PRODUCT IDENTIFICATION	                                    Revision Date:

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: Ultra Finish                 Product ID#:	 109650		HMIS SYMBOL     		HMIS   NFPA      		NFPA SYMBOL

Generic Name: Finish					                 HEALTH        | 1  |   	            SEVERE 4 EXTREME   		FIRE        | 0  |

Supplier's Name: BIRSCH INDUSTRIES, INC. 			FLAMMABILITY  | 0 |          SERIOUS 3 HIGH      		REACTIVITY  | 0  |

Supplier's Address: 476 Viking Drive, Suite 102 			REACTIVITY    | 0  | 	     MODERATE 2 MODERATE  	TOXICITY    | 1  |

Virginia Beach, VA  23452	  		               	                      SLIGHT 1 SLIGHT    		SPECIAL     | 0 |

Emergency Phone Number: (800) 255-3924				                       	          MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                      ------Exposure Limits-----       	  VP

ACGIG?TLV	OSHA/PEL		mmHG

Diethylene Glycol Methyl Ether			00011-77-3	<65%		.2000MMHG @ 68F		 NONE

*  These items are listed on the SARA TITLE III  Section 313 inventory.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                              		|                        SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212 F	       		Specific Gravity: 1.03		| Flash Point (Method Used): None

Vapor Pressure: N/A			Vapor Density:   (Air=1): N/A		| Upper Explosive Limit: N/A

% Volatile: 44-46 %	              		pH: 8.0-8.5			| Extinguishing Media: CO2, Water Mist, Dry Chemical, Foam	                  Solubility in Water: Yes	                                                     		|	Type BC or ABC

Evaporation Rate:(Water=1): <1						| Special Fire Fighting Procedures: None.

Physical Description: Milky white emulsion with mild odor                                                | Unusual Fire & Explosion Hazards: None

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------                  	        SECTION 5 - REACTIVE DATA                             		|                       SECTION 6 - STORAGE AND HANDLING INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------Stability: Stable         	Hazardous Polymerization: Will not occur			| Precautions to be Taken in Handling and Storage: Do not store or stack over 3 cases

Incompatibility (Materials to Avoid): None					|   or pails high

Hazardous Decomposition Products: None					|

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Eye or ingestion                                                 				|

Skin:	Redness and drying						| Skin: Wash thoroughly with water. If irritation persists, call physician

Eyes:	Redness and moderate irritation                                                      		| Eyes: Flush with water for 25 minutes. Call physician if irritation continues

Inhalation:                                                                                           		| Inhalation:

Ingestion: Slight mucosal damage along with stomach cramps, nausea		| Ingestion: Immediately drink large quantities of water. Contact a physician

And/or vomiting.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: None required					 Ventilation: Local exhaust

Protective Gloves: None		                                            		 Eye Protection: Protective goggles should be worn

Other Protective Equipment: Not required						|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN

THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled:			| Waste Disposal Method: Dispose in accordance with all Federal, State, and Local

Small spills -							| 	regulations.

Large spills -							|

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------