N/A = Not Applicable                                    		 MATERIAL SAFETY DATA SHEET                         		 Effective Date: Oct. 2002

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION			             Revised Date: Feb 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: UNMISTAKABLE GREEN GLASS CLNR       Product ID#: 178060      	HMIS SYMBOL     		HMIS      NFPA      		NFPA SYMBOL

Generic Name: Glass Cleaner              					HEALTH        |1|                      SEVERE 4 EXTREME   	FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|             SERIOUS 3 HIGH       		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102  				REACTIVITY    |0|            MODERATE 2 MODERATE  	TOXICITY    |1|

Virginia Beach, VA  23452						               SLIGHT 1 SLIGHT     		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                                                      MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       	CAS #           	WT. %                        TWA-TLV             	STEL-TLV                  CARCINOGEN

MG/M3               	MG/M3

Neutralized Dodecylbenzinesulfonic acid          	27176-87-0         	0-5.0                          	N/A                 	N/A                          	No

*MEA                                                		141-43-5         	0-2.0                          	7.5                 	15                           	No

DPM                                              		34590-94-8         	0-2.0                          	606                 	909                          	No

Nonylphenol Ethoxylate                           		26027-38-3         	0-5.0                         	N/A                 	N/A                          	No

Water                                             		7732-18-5        	to 100                         N/A                 	N/A                          	No

This product contains *toxic chemicals subject to

the reporting requirements of section 313 of the Emergency Planning

and Community Right-To-Know Act of 1988 and of 40 CFR 372.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                                      |                           SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): >212o F     		Specific Gravity: 1.032                    	| Flash Point (Method Used): None to boil

Vapor Pressure: N/A           		Vapor Density:      (Air=1): N/A           	| Upper Explosive Limit: None

% Volatile: N/A               		pH: 10.0-10.2                              		| Extinguishing Media: N/A

Solubility in Water: Yes      		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: None

Physical Description: Green liquid with a citrus odor                    			| Unusual Fire & Explosion Hazards: None

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                             	|                    SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Do not stack cases or pails

Incompatibility (Materials to Avoid): Do not mix with ANY other chemicals		|   over  five high.

Hazardous Decomposition Products: None                                   			|

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry: Skin						|

Skin: May produce irritation to skin.                                 			| Skin: Immediately wash with soap and plenty of water.

Eyes: Can cause irritation or burns.                                  			| Eyes: Immediately flush eyes with plenty of water for at least 15 minutes. Seek

|    medical aid.

Inhalation: Mists may produce respiratory irritation.                 			| Inhalation: Remove to fresh air Seek medical aid if irritation persists.

Ingestion: Can cause irritation and burns to mouth throat             			| Ingestion: DO NOT induce vomiting. Give large quantities of water.

and stomach.                                                            			|    Seek medical aid.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: None.                                            			| Ventilation: Local exhaust is recommended

Protective Gloves: Rubber                                                			| Eye Protection: Chemical splash goggles.

Other Protective Equipment: None                				|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Mop, vacuum or		| Waste Disposal Method: Dilute with plenty of water and flush into sewer system.

wash away with water.					|