N/A = Not Applicable                                      		      MATERIAL SAFETY DATA SHEET                          	                         Effective Date: Oct. 2002

-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

# SECTION 1 - PRODUCT IDENTIFICATION  		                            Revised Date: Feb. 2004

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Product Name: VAC-UP STRIPPER                      	Product ID#: 179545            	HMIS SYMBOL     		HMIS   NFPA      		NFPA

Generic Name: Stripper                                                         			HEALTH        |2|   	           SEVERE 4 EXTREME   		FIRE        |0|

Supplier's Name: BIRSCH INDUSTRIES, INC. 				FLAMMABILITY  |0|           SERIOUS 3 HIGH      		REACTIVITY  |0|

Supplier's Address: 476 Viking Drive, Suite 102 				REACTIVITY    |0| 	     MODERATE 2 MODERATE  	TOXICITY    |2|

Virginia Beach, VA  23452						              SLIGHT 1 SLIGHT    		SPECIAL     |0|

Emergency Phone Number: (800) 255-3924                                                                                                                     MINIMAL 0 INSIGNIFICANT

Information Phone Number: (757) 622-0355

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 2 - INGREDIENTS

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

CHEMICAL NAME                                       		CAS #           	WT. %                 	TWA-TLV             	STEL-TLV            CARCINOGEN

MG/M3               	MG/M3

Ethylene glycol monobutyl ether                   	                  	111-76-2         	20.0 to 35.0             	121                 	N/A                          	No

*Monoethanolamine                                  			141-43-5         	 8.0 to 12.0                	7.5                  	15                          	No

Sodium 1-Octane Sulfonate                         			5324-84-5         	 3.0 to 6.0                 	N/A                 	N/A                          	No

Water                                             			7732-18-5         	to 100                         N/A                 	N/A                          	No

These items are listed on the SARA TITLE III Section 313 inventory.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 3 - PHYSICAL DATA                	                 |                             SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boiling Point (F): 212oF      		Specific Gravity: 0.99                     	| Flash Point (Method Used): Above boiling point

Vapor Pressure: 20 mm Hg @68oF 	Vapor Density:      (Air=1): >1            	| Upper Explosive Limit: N/A

% Volatile: 80                		pH: 11.1-12.0                                  		| Extinguishing Media: CO2 or water

Solubility in Water: Complete 		Evaporation Rate:   (Water=1): <1          	| Special Fire Fighting Procedures: None

Physical Description: Blue watery thin liquid with a butyl smell.        		| Unusual Fire & Explosion Hazards:

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 5 - REACTIVE DATA                      		|        	       SECTION 6 - STORAGE AND HANDLING INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Stability: Stable          Hazardous Polymerization: Will not occur      		| Precautions to be Taken in Handling and Storage: Keep out of reach of children.

Incompatibility (Materials to Avoid): Do not mix with other chemicals.   		| For use by trained personnel only. Keep container closed during storage. For

Hazardous Decomposition Products: Alkaline vapors in a fire.             		| institutional and industrial use only.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 7 - HEALTH HAZARDS AND FIRST AID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Effects of Overexposure                                                  			| First Aid Procedures

Primary Route of Entry:						|

Skin: Severe skin irritant. May cause reddening, swelling, and        		| Skin: Flush with water. Seek medical attention if irritation persists.

possible skin damage.                                           			|

Eyes: Severe eye irritant. Liquid and mist may damage eyes, causing   		| Eyes: Flush with large quantities of water, holding eyelids open. Seek medical

corneal damage.                                                 			|       	attention if irritation persists.

Inhalation: Vapors and mist may be irritating to mucous membranes in  		| Inhalation: Get to fresh air. Seek medical attention if irritation persists.

the throat, and lungs.                                          			|

Ingestion: Irritating and corrosive to the mouth and throat. May      		| Ingestion: Do not induce vomiting. Drink large quantities of water. Seek

cause headache, nausea, abdominal pain, vomiting and diarrhea.  		|       	medical attention immediately.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 8 - SPECIAL PROTECTION INFORMATION

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Respiratory Protection: Use in well ventilation area.                    			| Ventilation: Provide local exhaust to keel TLV of Section 2 ingredients below

|       acceptable limit.

Protective Gloves: Waterproof                                            			| Eye Protection: Safety glasses.

Other Protection: Waterproof shoes   					|

PLEASE READ AND FOLLOW THE DIRECTIONS ON THE PRODUCT LABEL. THEY ARE YOUR BEST GUIDE FOR THIS PRODUCT IN THE MOST EFFECTIVE WAY, AND THEY ALSO GIVE THE NECESSARY SAFETY PRECAUTIONS TO PROTECT YOUR HEALTH.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SECTION 9 - SPILL OR LEAK PROCEDURES

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Steps to be Taken in Case Material is Released or Spilled: Dike and      		| Waste Disposal Method: Any method in accordance to applicable laws..

absorb spill with inert material and transfer to container for disposal. 		|

SECTION 10 – TRANSPORTATION INFORMATION

REGULATED BY DOT:  			Not Regulated

DOT PROPER SHIPPING NAME:		N/A

UN NUMBER:				N/A

HAZZARD CLASS:				N/A

PACKING GROUP:				N/A